|  |  |  |
| --- | --- | --- |

BT NETWORK INTERCONNECTION PROVISIONING MANUAL

APPENDIX 12 – Statement of Compliance to:

I/C Specifications 0100, 0120, 0200 and 0300

# CONTENTS

CONTENTS	1

1 INTRODUCTION	3

2 COMPLETING THE FORM	3

4 STATEMENT OF COMPLIANCE WITH  I/C SPECIFICATION 0100	5

Responsibilities	5

Relevant Parameters	5

Apportionment of Overall Loudness Rating (OLR)	5

Transmission Time	5

Echo Loss	5

Stability Loss	6

Quantising Distortion	6

Coding Standards	6

Noise	6

Attenuation Distortion	6

Group delay distortion	6

Error Performance	6

Non Speech Levels	7

GSM/PCN Headset and Handset Sensitivity/Frequency Characteristics	7

5 STATEMENT OF COMPLIANCE WITH I/C SPECIFICATION 0120	8

6 STATEMENT OF COMPLIANCE WITH I/C SPECIFICATION 0200	9

Physical interface for Customer sited interconnect	9

System Synchronisation	10

Functional Characteristics of the Interface	10

Safety and Protection	11

7 STATEMENT OF COMPLIANCE WITH I/C SPECIFICATION 0300	12

Message Transfer Part	12

User Part	12

Interworking	12

Services	13

Testing	13

8 DOCUMENT HISTORY	14

## 1 INTRODUCTION

BT Wholesale requires the Communications Provider (CP) to make Statements of Compliance with the Interconnect Specifications 0100, 0120, 0200, 0300.  In order to standardise the response format, this document should be completed by CPs with reference to the above specifications:

- When applying to Interconnect with the BT network for the first time;

- When applying to Interconnect with a switch not listed in Appendix 1 of the BT (PSTN) Interconnect Provisioning Manual.

Note: the Interconnect Specifications1 and Appendix 12 are available from the “Pricing and Contracts” section of the BTWholesale.com website

1 www.BTWholesale.com – pricing & contracts – contractual information – telephony reference offer

2 www.BTWholesale.com – pricing & contracts – network resources – manuals – PSTN provisioning manual

## 2 COMPLETING THE FORM

This form should be downloaded and completed off-line.

Please confirm compliance with an “x” in the check box.  (To complete electronically, click on the check box).

If the CP/switch is not fully compliant to each statement, please leave the check box blank and explain why in the comment box.

The completed form should be submitted to your BT Technical Account Manager.

3 COMMUNICATIONS PROVIDER(CP) DETAILS

Please provide the following information:

Date:

CP Name:

CP Contact Name:

CP Contact E-mail address:

CP Contact Telephone number:

Switch Supplier:

Switch Type:

Switch Hardware Build:

Switch Software Build:

Any further information you wish to add:

## 4 STATEMENT OF COMPLIANCE WITH  I/C SPECIFICATION 0100

### Responsibilities

Confirm that the Operator will take overall responsibility for transmission quality on calls using indirect access services.

Comments:

### Relevant Parameters

Confirm that the Operator shall work with BT to endeavour to achieve requirements and objectives for overall loss, delay, echo loss, quantising distortion and coding standards on live site interconnects.

Comments:

Confirm that the Operator shall plan in accordance with guidance given for the remaining parameters listed.

Comments:

### Apportionment of Overall Loudness Rating (OLR)

Please indicate the type of Operator system transmission path – mark one only:

Analogue ;		Mixed Analogue/Digital ;			Digital

Confirm that the Operator shall work with BT to endeavour to provide end-to-end connections which fall within overall loudness ratings in accordance with the recommendations.

Comments:

### Transmission Time

Confirm that the Operator system transmission delays are in accordance with the recommendations.

Comments:

### Echo Loss

Confirm that the Operator system complies with the recommendations with regard to Echo Loss.

Comments:

### Stability Loss

Confirm that the Operator system complies with the recommendations with regard to Stability Loss.

Comments:

### Quantising Distortion

Confirm that the Operator system complies with the recommendations with regard to Quantising Distortion.

Comments:

### Coding Standards

Confirm that the Operator system complies with the recommendations with regard to Coding Standards.

Comments:

### Noise

Confirm that the Operator system complies with the recommendations with regard to Noise.

Comments:

### Attenuation Distortion

Confirm that the Operator system complies with the recommendations with regard to Attenuation Distortion.

Comments:

### Group delay distortion

Confirm that the Operator system complies with the recommendations with regard to Group Delay Distortion.

Comments:

### Error Performance

Confirm that the Operator system complies with the recommendations with regard to Error Performance.

Comments:

### Non Speech Levels

Confirm that the Operator system complies with the recommendations with regard to Non Speech Levels.

Comments:

### GSM/PCN Headset and Handset Sensitivity/Frequency Characteristics

If appropriate, confirm that the Operator system complies with the recommendations with regard to GSM/PCN Headset and Handset Sensitivity/Frequency Characteristics.

Comments:

End of Statement of Compliance with I/C spec 0100

## 5 STATEMENT OF COMPLIANCE WITH I/C SPECIFICATION 0120

Please state the type of interface that will be used to interconnect with the BT Wholesale network – mark one only:

2Mbit/s ;		STM-1	;		STM-4

If STM-n, confirm that the Operator system complies with the “Technical Recommendations” agreed in the Public Network Interest Group relating to STM-1 and STM-4.

Comments:

End of Statement of Compliance with I/C spec 0120

## 6 STATEMENT OF COMPLIANCE WITH I/C SPECIFICATION 0200

### Physical interface for Customer sited interconnect

Complete section a) or b) as appropriate

#### 2Mbit/s presented  interconnect

Confirm that the Operator system shall provide a 75 ohm coaxial connection compliant with G703 §6.1, 6.2, 6.3

Comments:

Earthing of screen

Confirm that the output port screen shall be bonded directly to the equipment metalwork at the equipment boundary (or as near as practical), and that the input port screen shall be earthed via a capacitor (typically 0.1µF) with an option for a DC earth connection if required.

Comments:

Interference

Confirm that the input ports shall tolerate, without error, interference from a non-synchronous standard test signal at a level 18dB lower than the wanted signal.

Comments:

Jitter/wander

Confirm that the output jitter shall not exceed 0.05UI under worst case operating conditions when measured in the frequency range 20Hz to100kHz, and that the tolerance of the input port to jitter/wander complies with ITU-T G.823 §3.1.1

Comments:

#### Aggregate interconnect

Confirm that the operator shall provide an STM-1 or STM-4 using

Either:

An electrical interface using 75 ohm coaxial connection compliant with G703:

Or:

Optical fibres with an FC type optical connector at the BT end:

Comments:

### System Synchronisation

Please state whether the Operator system shall obtain synchronisation from:

- A traceable source compliant with ITU-T recommendation G.811

- The BT system via a suitable 2Mbit/s link

Confirm that, if taking sync from the BT system, failure of the signalling link will result in the Operator system automatically taking sync from an alternative link (if available).

Comments:

Confirm that if sync is not available from any link, that the Operator system will enter a holdover mode keeping within the operation limits specified in ITU-T recommendation G.812 §2.2.3.

Comments:

### Functional Characteristics of the Interface

Confirm that the Operator system shall conform to the functional characteristics of the 2Mbit/s interface in accordance with the ITU-T recommendations G.704 (synchronous frame structures) and G.706 (frame alignment and CRC procedures).

Comments:

Alarm Indication

Confirm that the Operator system shall detect AIS (indicated by a continuous stream of binary “1”s) in the presence of an error ratio of 1 in 1000, and that all bits except the frame alignment word in the “1” state will not be mistaken as an AIS.

Comments:

Channel timeslot encoding

Confirm that the bearer channels shall carry “A” law encoded information as defined in ITU-T recommendation G.711

Comments:

Confirm the idle bit pattern transmitted is in accordance with ITU-T recommendation Q.522 §2.12

Comments:

### Safety and Protection

Dangerous Voltages

Confirm that where the Operator equipment uses or generates excessive voltages as defined in BS 6301:1989, that the interface with the BT network shall be electrically isolated from those voltages using suitable devices (as described in BS 6301:1989).

Comments:

Radiation Hazards

Confirm that where radio equipment is used, that arrangements shall be made to protect all personnel from levels of radiation exceeding 1 milliwatt per square centimetre.

Comments:

End of Statement of Compliance with I/C spec 0200

## 7 STATEMENT OF COMPLIANCE WITH I/C SPECIFICATION 0300

### Message Transfer Part

Confirm that the Operator system when interconnected to the BT network in the national signalling domain shall comply with the specification of the SS7 Message Transfer Part as defined in the current issue of NICC document ND1005 (PNO-ISC/SPEC/005).

Comments:

### User Part

Complete sections a), b), c) as appropriate:

- IUP (with or without Enveloped ISUP)

  - Confirm that the Operator system shall comply with the IUP specification as defined in the current issue of NICC document ND1006 (PNO-ISC/SPEC/006).

  - Confirm that where the Operator system supports proprietary extensions to IUP, that it shall comply with appropriate sections of  the current issue of NICC document ND1104 (PNO-ISC/INFO/004).

Comments:

- UK ISUP

  - Confirm that the Operator system shall, as a minimum, comply with the UK ISUP specification as defined in the current issue of NICC document ND1007 (PNO-ISC/SPEC/007).

  - Confirm that where the Operator system does not support a particular ISUP message, parameter or value, that the ISUP compatibility procedures shall be invoked (unless otherwise agreed) as defined in the current issue of NICC document ND1007 (PNO-ISC/SPEC/007).

Comments:

- SCCP

Confirm that the Operator system shall comply with the connectionless signalling protocols of the SCCP as defined in ITU-T recommendations Q.711-714 inclusive.

Comments:

### Interworking

Confirm that where the Operator system interworks:

- IUP with UK ISUP signalling protocol, this shall be in accordance with the current issue of NICC document ND1008 (PNO-ISC/SPEC/008).

- “IUP with Enveloped ISUP” with UK ISUP signalling protocol, this shall be as defined in the current issue of NICC document ND1104 §6 (PNO-ISC/INFO/004 §6).

Comments:

### Services

- Confirm that the Operator shall inform BT of any changes to the interface, services or features.

- Subject to section 7 items a) and b) in the Interconnect specification 0300, Confirm that the Operator shall not invoke services and/or features not included in the supplement to Interconnect specification 0300.

Comments:

### Testing

Confirm that where the Operator plans to enhance the interface by adding a service or feature after interconnection has been provided, that the Operator shall inform BT, provide details of the changes and agree any additional testing which may be deemed necessary.

Comments:

End of Statement of Compliance with I/C spec 0300

## 8 DOCUMENT HISTORY

| ISSUE No. | DATE | AUTHOR | SUMMARY OF CHANGES |
| --- | --- | --- | --- |
| 1.0 | 15/11/2005 | Roger Felgate | Initial Issue |