Issue F

March 2003

GENERIC

SIGNALLING SYSTEM No.7

SPECIFICATION

I/C Specification 0300

CONTENTS

1.	SCOPE	3

2.	REFERENCES	3

3.	OVERVIEW	3

4.	PARTS OF SIGNALLING SYSTEM No.7	5

4.1  MESSAGE TRANSFER PART (MANDATORY)	5

4.2  	USER PARTS	5

4.2.1 	INTERCONNECT USER PART with or without Enveloped ISUP (OPTION)	5

4.2.2 	UK ISUP  (OPTION)	5

4.3  	SIGNALLING CONNECTION CONTROL PART (OPTION)	6

5. 	AVAILABILITY	7

6.	INTERWORKING	7

7.	SERVICES	8

8.	TESTING	8

9.	GLOSSARY	9

10.     HISTORY	9

APPENDIX 1 - SIGNALLING INTERCONNECT OPTIONS	10

GENERIC SIGNALLING SYSTEM No.7 SPECIFICATIONS

# 1.	SCOPE

This document describes the interconnect between the BT System and the Operator System within the UK.

# 2.	REFERENCES

All references contained in this document are correct at time of issue. Bilateral agreement may be required before any additional functionality, included in later editions of these specifications, are used over a point of interconnect.

At the time of issue of this document, NICC/PNO-ISC documentation is available on the OFTEL website.

# 3.	OVERVIEW

- The ITU-T common channel signalling system number seven (SSNo.7 or C7) shall be used as the signalling system for the conveyance of PSTN and ISDN telephony calls between the Operator System and the BT System. (The available Parts of the signalling System are more fully defined in section 4).

- BT and the Operator shall support the Interconnect MTP.

- BT and the Operator shall support at least one User Part on each Interconnect Route. The User Parts at each end of the signalling link shall be compatible.

- The interconnect schematic of BT’s Signalling System No.7 (SSNo.7) is shown in Figure 1.

Note:

International switches do not recognise or generate UK parameters, and use the compatibility mechanisms to allow the call to continue.

UK Network

BT Network

IUP

Env ISUP

UK ISUP

IUP

UK ISUP (note)

SCCP

IUP

Env ISUP

UK ISUP

Trunk

International

IUP

Env ISUP

UK ISUP

Local

STP

Interconnect

MTP

Interconnect

MTP

Interconnect

MTP

Interconnect

MTP

Figure 1 - Signalling System No.7 –Network Schematic

- The conveyance of PSTN and ISDN calls between the Operator System and the BT System within the UK shall use the Interconnect MTP and either IUP or UK-ISUP as specified by NICC/PNO-ISC, unless otherwise agreed between the Operator and BT.

- For the conveyance of Connectionless Services in support of mobile telephony, the BT System and Operator System may agree to use the SCCP and Interconnect MTP.

- The Operator and BT shall agree which services and features listed in the supplement to this Agreement shall be supported across the interface before BT shall be obliged to convey SSNo.7 signalling messages.

# 4.	PARTS OF SIGNALLING SYSTEM No.7

## 4.1  MESSAGE TRANSFER PART (MANDATORY)

The BT System and Operator System when interconnected in the national signalling domain shall comply with the specification of the SSNo.7 Message Transfer Part (MTP) as defined in NICC Document ND1005:2000/08 (PNO-ISC/SPEC/005 Issue 3).

## 4.2  	USER PARTS

- Subject to availability, the Operator and BT shall, unless otherwise agreed, use at least one of the User Parts, defined further in the following sub-sections.  The generic availability of these User Parts on the various BT System Types is provided in Section 5.

- Subject to availability, the Operator and BT may agree to support a combination of User Parts between their Systems. The combinations that can be offered on a route-by-route basis are defined in Appendix 1 to this specification.

### 4.2.1 	INTERCONNECT USER PART with or without Enveloped ISUP (OPTION)

The BT System and Operator System shall comply with the specification of the SSNo.7 Interconnect User Part as defined in NICC Document ND1006:2001/06 (PNO-ISC/SPEC/006 Issue 4), with the following additions:

- Where the BT System and Operator System support propriety extensions to IUP they shall comply with the appropriate sections of NICC Document ND1104:1999/10 (PNO-ISC/INFO/004 Issue 2) provided that a bilateral agreement exists between BT and the Operator to use these extensions.

- If the BT or Operator System does not support a propriety extension to IUP, that System shall respond in accordance with NICC Document ND1006:2001/06 (PNO-ISC/SPEC/006 Issue 4).

- The national and international prefixes shall be conveyed in the called address, where appropriate.

### 4.2.2 	UK ISUP  (OPTION)

- The BT System and Operator System shall comply with the specification of the SSNo.7 ISDN User Part, as specified in NICC Document ND1007:1999/03 (PNO-ISC/SPEC/007 Issue 2.2), in accordance with the industry UK Interconnect Signalling MOU. Functionality in addition to the UK Interconnect Signalling MOU will require Bilateral Agreement.

- Where the Operator System or BT System does not support a particular ISUP message, parameter or value, the ISUP compatibility procedures shall be invoked i.e. unless otherwise agreed, the Operator and BT shall use the ISUP compatibility mechanism to ensure that different capabilities in one System do not adversely affect the other Systems Capabilities in accordance with NICC Document ND1007:1999/03 (PNO-ISC/SPEC/007 Issue 2.2).

Note:

UK NICC/PNO-ISC have identified that the specification of ISUP congestion control is incomplete and this may result in the congestion control being ineffective. PNO-ISC and PNO-IG have approved a revised UK ISUP Specification, NICC Document ND1007:2001/07 (PNO-ISC/SPEC/007 issue 3), which defines how an adaptive ACC implementation shall behave. Until most UK PNOs deploy switches with the new adaptive ACC, discussions will be required between BT and any interconnecting operators to ensure that the congestion control implementations are appropriate for the interconnect. To enable the interconnect to proceed these discussions must result in an agreement of the controls to be used. PNO-ISC has approved guidelines for such discussions. For full details see section 8 of NICC Document ND1115:2001/08 (PNO-ISC/INFO/015 Issue 1) ‘ISUP Overload Controls’.

## 4.3  	SIGNALLING CONNECTION CONTROL PART (OPTION)

Where the BT System and a UK GSM Mobile Operator System have agreed the support of Non-Circuit Related Signalling (NCRS) services to enable Global Systems for Mobile (GSM) telephony users to have GSM Roaming Services when travelling abroad, they shall comply with Connectionless Signalling Protocols of the Signalling Connection Control Part as defined ITU-T recommendations Q.711-714 (inclusive).

5. 	AVAILABILITY

| BT System Type | Interconnect MTP |  |  |  |
| --- | --- | --- | --- | --- |
|  | IUP | Enveloped ISUP | UK ISUP | SCCP |
| Local | Yes | Yes | Yes | No |
| Trunk | Yes | Yes | Yes | No |
| STP | Yes | Yes | Yes | No |
| International | Yes | No | Yes (Note) | Yes |

Note:

International switches do not recognise or generate UK parameters and use the compatibility mechanisms to allow the call to continue.

Table 1 – SSNo.7 Signalling Parts available at various BT System Types.

- The SSNo.7 MTP and chosen User Parts(s) shall be used between the Operator system and the BT system until such time as agreed by both the Operator and BT, or as otherwise lawfully directed by the Director General of Oftel, or the European Commission.

- BT shall advise the Operator if interconnection of a particular User Part is not available at a specific Point of Connection, for example if due to operational constraints.

# 6.	INTERWORKING

- The Operator and BT shall interwork the UK ISUP and IUP signalling systems as necessary in accordance with NICC Document ND1008:1999/03 (PNO-ISC/SPEC/008 Issue 1), where the conveyance of the service or feature defined is compatible on each of the User Parts and agreed between Operators. BT and the other PNO may be required to state if UK ISUP will be transported transparently across their network.

- The Operator and BT shall interwork “IUP with enveloped ISUP” with the UK ISUP signalling system as defined in Section 6 of NICC Document ND1104:1999/10 (PNO-ISC/INFO/004 Issue 2).

- The number of different versions and implementations of the signalling system makes the delivery of end-to-end services and features complex. BT remains responsible for the conveyance of Signalling Messages within the BT System. BT can have no responsibility or liability for the acts or omissions of any third party telecommunications operator or overseas administration in respect of any signalling message handed over to or received from such third party. BT cannot guarantee that all services and features that transit the BT System will interwork satisfactorily end-to-end.

- Additionally, international services and features that transit the BT System may be limited by the terms of agreements between BT and an Authorised Overseas Administration, which may be applied to maintain the technical viability of the international interconnect or be a regulatory requirement imposed on the Authorised Overseas Administration’s network.

- BT and the Operator shall co-operate in accordance with Annex A of this Agreement to minimise any service disruption that may arise, for example due to signalling system incompatibilities.

# 7.	SERVICES

- The Supplement to this specification lists the schedules that can be supported and the services and features available on each of the User Parts listed in section 4. Initially some of the services shown may not be supported on the BT System. BT intends to make these services available as the capability is developed on the BT System.

- The services available may vary from those identified in the supplement depending on whether the BT System or the Operator System is originating, terminating or transiting a particular service schedule.

- The Operator and BT shall inform each other of changes to the interface, services or features according to the terms and conditions defined in this Agreement.

- Subject to items a) and b) above, the BT System shall support the list of services and/or features identified in the supplement to this Agreement.

- Subject to items a) and b) above, BT may invoke any of the services or features identified for a particular User Part in the supplement to this Specification.

- Subject to items a) and b) above, BT and the Operator shall not invoke services and/or features not included in the supplement to this Specification, for a particular User Part.

# 8.	TESTING

- The Operator and BT shall confirm in writing that their implementation conforms to the relevant specifications and identify any deviations from that specification to be agreed between the parties before BT shall be obliged to convey SSNo.7 signalling messages.

- BT and the Operator agree to test the SSNo.7 signalling interface in accordance with Annex A of this Agreement.

- The testing shall be completed to both Parties’ satisfaction before BT shall be obliged to convey SSNo.7 signalling messages.

- Where the Operator or BT enhance the interface by adding a service or feature after interconnection has been provided, the Parties shall inform each other, provide details of the changes and agree any additional testing that may be deemed necessary in accordance with the terms and conditions in this Agreement.

# GLOSSARY

ACC	Automatic Congestion Control

DISC	Digital International Switching Centre

GSM	Global System for Mobile

ISDN	Integrated Services Digital Network

ISUP	ISDN User Part

ITU	International Telecommunication Union

IUP	Interconnect User Part

MOU	Memorandum Of Understanding

MTP	Message Transfer Part

NCRS	Non-Circuit Related Signalling

NICC 	Network Interoperability Consultative Committee

PNO	Public Network Operator

PNO-IG	Public Network Operators- Interest group

PNO-ISC	Public Network Operators - Interconnect Standards Committee

PSTN	Public Switched Telephony Network

SCCP	Signalling Connection Control Part

SS	Signalling System

STP	Signalling Transfer Point

# 10.     HISTORY

Issue 1.0	January 1994; Issue 1.1	February 1994; Issue 1.2 April 1994

Issue A	August 1994

Issue B	August 1994

Issue C	January 1996

Issue D	January 1998

Issue E	June 1998

Issue F	March 2003

# APPENDIX 1 - SIGNALLING INTERCONNECT OPTIONS

Note:

Only available at DISCs

Operator System

BT

System

IUP Signalling Route

IUP Traffic Route

Operator System

BT

System

IUP with enveloped ISUP Signalling Route

IUP with enveloped ISUP Traffic Route

Operator System

BT

System

UK ISUP Signalling Route

UK ISUP Traffic Route

Shared IUP/UK ISUP Signalling Route

IUP Traffic Route

UK ISUP Traffic Route

Operator System

BT

System

Shared IUP with enveloped ISUP/UK ISUP Signalling Route

IUP with enveloped ISUP Traffic Route

UK ISUP Traffic Route

Operator System

BT

System

UK ISUP Signalling Route

UK ISUP Traffic Route

IUP Traffic Route

Operator System

BT

System

IUP Signalling Route

UK ISUP Signalling Route

UK ISUP Traffic Route

IUP with enveloped ISUP Traffic Route

Operator System

BT

System

IUP with enveloped ISUP Signalling Route

SCCP Signalling Route (Note)

Operator System

BT

System

Plus Either Option 1, 3.or 4

1.

2.

3.

4.

5.

6.

7.

8.