FRAME RELAY

###### TECHNICAL SPECIFICATION

Frame Relay I/C Specification

### 1.0  INTRODUCTION

### This specification describes how Frame Relay works and identifies the main international standards which apply to the BT interconnect product

### 1.1 Frame Relay Frame Format

The following illustrates the format of a frame relay frame.

Frame Relay Frame Format

As the illustration shows, the frame relay frame is very similar to the other High-level Data Link Control (HDLC) frames. It is bounded by flags, has a 2 octet Frame Check Sequence (FCS), and a variable length user data field. The frame relay header is often referred to as the address field.

The header contains the following bit fields:

- Data Link Connection Identifier (DLCI)

- Command/Response (C/R)

- Forward Explicit Congestion Notification (FECN)

- Backward Explicit Congestion Notification (BECN)

- Discard Eligibility (DE)

- Extended Address (EA)

#### 1.1.1 Data Link Connection Identifier (DLCI )

The DLCI is the addressing portion of a frame relay frame. The DLCI identifies a single connection (virtual circuit) corresponding to a particular destination. The length of the header, from 2 to 4 octets, is specified using the EA bit. The 2 octet length is the default. When the header is 2 octets, the DLCI is 10 bits. The valid range of DLCI values is shown in the Table  below:

.

| Range | Usage |
| --- | --- |
| 0 | ANSI T1.617 Annex D and CCITT Q.933 Annex A Local In-channel Signalling |
| 1-15 | Reserved |
| 16-991 | Assignable to user frame relay logical links |
| 992-1007 | Reserved |
| 1008-1023 | Reserved |

#### 1.1.2 Command/Response (C/R)

The C/R bit is not used by the network. This bit is passed transparently between users. The coding of the C/R bit is application specific. This bit is typically used in data link protocols to indicate whether the frame is a command or response

1.1.3 Forward Explicit Congestion Notification (FECN)/ Backward Explicit Congestion Notification (BECN)

The FECN and BECN bits are used to signal to source and destination user devises that network congestion is occurring. FECN bits are used to notify the data destination. BECN bits are used to notify the data source.

The method of responding to congestion notification is dependent on the user’s protocols and flow control mechanism. The user may make use of either the FECN or BECN bit to react to congestion. If the user does not react to congestion notification, the network may be required to discard the user’s frames in order to alleviate congestion. If the user responds to congestion notification by reducing the flow of traffic, frames may still be discarded, although discard is less likely.

#### 1.1.4 Extended Address (EA )

The EA bit allows the length of the header to be extended from 2 to 3 or 4 octets. The first bit of each octet is reserved to designate whether or not that octet is the last octet of the address field. A 0 indicates another octet of address follows. A 1 indicates that the current octet is the last octet of the address field.

1.1.5 Discard Eligibility (DE)

The DE (bit is set to 1 by either the user or network to indicate that a frame should be discarded in preference to other frames in a congestion situation. Frames with the DE bit set to 0 are less likely to be discarded, but are still subject to discard when the network is congested.

Setting of this bit by the user is optional. The network will not clear the DE bit upon exit from the network.

#### 1.1.6 Frame Relay Information Field

The maximum length of a frame relay information field is 8187 octets, although a maximum size of 4096 is recommended. The network is not responsible for detecting errors in frames exceeding 4096 octets since FCS is only effective in detecting errors in frames up to 4096 octets.

#### 1.1.8 Frame Check Sequence (FCS)

The FCS field is the standard 16 bit cyclic redundancy check used for HDLC frames. The FCS is used to detect bit errors occurring in the bits of the frame between the opening flag and the FCS.

1.2 .1 Permanent Virtual Circuit (PVC)

PVCs are static connections between two points that need to communicate with one another. This means the virtual circuit is hard-coded or entered in tables at the network devices providing the connection. These circuits are always considered by the end-point devices “connected.” They are built when the device is brought on-line and they are never released even if the connection is inactive or not transmitting data. CCITT Q.933 Annex A, T1.617 Annex D or LMI are used to manage PVCs.

### 1.2.2  Switched Virtual Circuit (SVC)

When implemented, SVCs for frame relay will conform to CCITT Q.933. SVCs are set up dynamically on a call-by-call basis. This means the user can dynamically request a connection to a particular destination. When the connection is no longer needed, it is cleared.

SVCs are currently being implemented by frame relay switch vendors.

### 1.2.3 Local In-channel Signalling

The local In-channel Signalling protocol is used to provide the access procedures for the detection and notification of the following conditions:

- Addition of a PVC

- Deletion of a PVC

- Availability (active state) of a configured PVC

- Unavailability (inactive state) of a configured PVC

- Local In-channel Signalling link reliability errors

- Local In-channel Signalling link protocol errors

- Local In-channel Signalling link sequence number errors

Two standards based protocols are provided for local In-channel signalling. The ANSI T1.617 Annex D and CCITT Q.933 Annex A are very similar to each other. The key differences between these standards are summarized below.

| Information Element | T1.617 Annex D | Q.933 Annex A |
| --- | --- | --- |
| Locking Shift | yes | no |
| Report type identifier code | 0000 0001 | 0101 0001 |
| Link integrity identifier code | 0000 0011 | 0101 0011 |
| PVC status identifier code | 0000 0111 | 0101 0111 |

Routers periodically issue Status Enquiry messages requesting Full Status Reports determining when a PVC has become active or inactive. Some LAN routers use OSPF or ARP to determine the LAN addresses and topology of the other end of the PVCs, thus allowing LAN routers to be frame relay network configuration insensitive. This process allows the routers to treat the frame relay PVCs as if they were “virtual private lines”.

### Committed Information Rate (CIR)

CIR is defined by T1.606 as the rate at which the network agrees to transfer information under normal conditions. This rate is measured over the measurement interval Tc and can be defined by the equation CIR = Bc/Tc, where Bc is a committed number of bits to be transmitted over an unspecified period of time Tc.  CIR is negotiated at service subscription time.

If during any measurement interval Tc transmission exceeds Bc, then the total excess is referred to as Be and those frames marked Discard Eligible (DE). Frames can be also marked by the user and are treated as Be traffic. During congestion, these frames with DE bit set to 1 will be discarded first.

The following list summarizes the standard service parameters and their definition.

Committed Burst Size (Bc) - the maximum amount of subscriber data that the network agrees to transfer under normal conditions during a time interval Tc.

Committed Information Rate (CIR) - the average throughput that the network commits under normal network conditions.

Access Rate (AR) - the rate at which user data instantaneously arrives at the access interface.

Excess Burst Size (Be) - the maximum amount of uncommitted data in excess of Bc that the network will attempt to deliver during a time interval Tc.

The Frame Relay Network to Network Interface (NNI) is an industry standard for interconnecting multi-vendor Frame Relay Systems.  BT's implementation aims to be fully compliant with the ITU-T Specification “Frame Relay Network-to-Network Interface Implementation Agreement Version 2.1, FRF2.1 dated 10 July 1995”which conforms to CCITT Q933A Annex A  & ANSI T1.617A Annex D international standards. This industry standard allows BT's Frame Relay System to be interconnected to other public Frame Relay Systems.