Issue G

April 2000

GENERIC

ELECTRICAL & PHYSICAL

INTERFACE SPECIFICATION

I/C Specification 0200

GENERIC ELECTRICAL & PHYSICAL INTERFACE SPECIFICATION

CONTENTS

1.	GENERAL	3

2.	PHYSICAL INTERFACE FOR CUSTOMER SITED INTERCONNECT	3

2.1	2M/bit/s Presented Customer Sited Interconnect	3

2.2	Customer Sited Interconnect Aggregate	4

3.	ELECTRICAL INTERFACE	4

3.1.	General Characteristics	5

3.2.	Specifications at the output ports	5

3.3.	Specifications at the input ports	5

3.4.	Earthing of screen	5

3.4.1.	Output Ports	5

3.4.2.	Input Ports	5

3.5.	Interference	5

3.6.	Jitter	5

3.6.1.	 Maximum jitter at output ports	5

3.6.2.	Tolerance of input ports to jitter and wander	6

3.6.3.	Measurement of Jitter	6

3.7.	Wander at BT and Operator input ports	6

4.	BT AND OPERATOR SYSTEM SYNCHRONISATION	6

4.1.	Operator System Synchronisation	6

4.2.	Deriving synchronisation from the BT System	7

5.	FUNCTIONAL CHARACTERISTICS OF THE INTERFACE	7

5.1.	Signalling	7

5.2.	Timeslot '0'	7

5.3.	Alarm Indication Signal (AIS)	8

5.4.	Channel Time Slot Encoding	8

6.	SAFETY AND PROTECTION	8

6.1.	Dangerous Voltages	8

6.2.	Radiation Hazards	8

7.	REFERENCES	9

8.	GLOSSARY	9

9	HISTORY	10

# 1.	GENERAL

This document defines the physical and electrical characteristics of Interconnect Links, between the BT System and the Operator System.  All sections apply with the exception of sections 2 and 3 which only apply to Customer Sited Interconnect (CSI) Interconnect Links.

All references to ITU Recommendations refer to the White Book unless otherwise indicated.

# 2.	PHYSICAL INTERFACE FOR CUSTOMER SITED INTERCONNECT

The interconnection between the BT System and the Operator System shall be provided by a BT digital path that terminates on a BT Circuit Termination Unit (CTU) located within the building housing the Operator Switch, the Operator Digital Distribution Frame (DDF) or Operator Equipment Optical Interface.

## 2.1	2M/bit/s Presented Customer Sited Interconnect

The BT CTU will present a G703 interface via two 75 ohm coaxial cables direct to either the Operator Switch or an Operator Digital Distribution Frame (DDF). The Point of Connection shall be the 75 ohm G703 coaxial connector at the BT end of the cables connecting the CTU to the Operator Switch or the Operator DDF (See Fig 1). The coaxial cables connecting the CTU to the Operator Switch shall have a loss not exceeding 6dB at 1024 kHz (see Fig 1). The cables connecting the BT CTU to the Operator System shall be provided and maintained by the Operator.

## 2.2	Customer Sited Interconnect Aggregate

The BT CTU shall present either an electrical or optical STM-1 or STM-4 interface which shall conform to the Technical Recommendation Specified in the Generic Synchronous Digital Hierarchy Interface Specification 0120.

The BT CTU Electrical interface shall be presented via two 75 ohm coaxial cables direct to either the Operator Switch or an Operator Digital Distribution Frame (DDF). The Point of Connection shall be the 75 ohm G703 coaxial connector at the BT end of the cables connecting the CTU to the Operator Switch or the Operator Digital Distribution Frame (See Fig 1).

The  BT CTU Optical Interface shall be presented via two Optical fibres direct to the Operator Equipment Optical Interface. The Point of Connection shall be the FC type Optical connector at the BT end of the fibres connecting the CTU to the Operator Equipment Optical Interface (See Fig 2).

The  cables and Optical fibres connecting the BT CTU to the Operator System shall be provided and maintained by the Operator.

# 3.	ELECTRICAL INTERFACE

The electrical interface for CSI Aggregate shall conform to the Technical Recommendation Specified in the Generic Synchronous Digital Hierarchy Interface Specification 0120.

The following section shall apply to a 2Mbit/s electrical interface using the coaxial pair option of ITU-T Recommendation G.703 (Physical/Electrical Characteristics of Hierarchical Digital Exchanges).

## 3.1.	General Characteristics

These shall conform with section 6.1 of ITU-T Rec. G703

## 3.2.	Specifications at the output ports

These shall conform with section 6.2 of ITU-T Rec. G.703 (Table 6).

## 3.3.	Specifications at the input ports

These shall conform with section 6.3 of ITU-T Recommendation  G.703.

## 3.4.	Earthing of screen

### 3.4.1.	Output Ports

At output ports the cable screen shall be bonded to the equipment metalwork at the equipment boundary or as near as possible to it.

### 3.4.2.	Input Ports

The input port cable screen shall be earthed via a capacitor (typically 0.1 mF) to the equipment. Provision shall be also made at this point for providing a DC connection to earth. The equipment shall be set-up with the DC earth not connected, this is illustrated in figure 1.

A suitable ferrite tube ferrule should be threaded onto the cable so as to be located at a point between the bonding point and the equipment circuitry

## 3.5.	Interference

The input ports shall tolerate, without error, interference from a non synchronous standard test signal (ITU-T Recommendation 0.151- Error Performance Measuring Equipment for Digital Systems At The Primary Bit Rate and Above) at a level 18dB lower than the wanted signal.

## 3.6.	Jitter

### 3.6.1.	 Maximum jitter at output ports

Under worst case operating conditions (i.e. fault free) the output jitter shall not exceed 0.05 UI when measured in the frequency range 20 Hz to 100 kHz.

Note: This assumes that the Operator Switch meets:

1.	the input jitter tolerances given in section 3.6.2.

2.	the jitter transfer function given in Figure 5 of ITU-T Recommendation Q.551 (Transmission Characteristics of Digital Exchanges).

### 3.6.2.	Tolerance of input ports to jitter and wander

The tolerance of both the BT and the Operator input ports to jitter shall be as defined in section 3.1.1 of ITU-T Recommendation G.823 (Jitter And Wander Tolerance of Digital Input Ports).

### 3.6.3.	Measurement of Jitter

A jitter measuring set conforming to the requirements of ITU-T Recommendation O.171 (Timing Jitter Measuring Equipment for Digital Systems) shall be used. BT and the Operator shall co-operate in the application of testing methods as described in ITU-T Recommendation G.823 (The Control of Jitter and Wander Within Digital Networks Which Are Based on the 2048 kbit/s Hierarchy).

## 3.7.	Wander at BT and Operator input ports

The tolerance of the BT and Operator input ports to wander shall be as defined in section 3.1.1 of ITU-T Recommendation G.823 .

# 4.	BT AND OPERATOR SYSTEM SYNCHRONISATION

## 4.1.	Operator System Synchronisation

To ensure synchronisation with the BT System, the Operator System shall employ a synchronisation system which is time traceable to a source complying with the requirements of ITU-T recommendations G.811.

The performance of digital clocks, which derive shall comply with the objective slip rate characteristics given in ITU-T recommendations G.811 and G.822 for the purpose of minimising timing perturbations in general and slip rates in particular.

## 4.2.	Deriving synchronisation from the BT System

If the Operator System is to derive synchronisation from the BT System it shall take its timing from BT nominated synchronisation feeds in a master/slave relationship.

The BT System employs a central master clock to maintain a co-operatively synchronised system within ITU-T recommended frequency limits.

The synchronisation utilities that co-operate to establish the synchronous clock rate are geographically located to ensure that any point in the BT System is contained within an 18 microsecond phase deviation (wander).

Where suitable, the synchronisation feeds may be taken from 2Mbit/s Interconnection Links carrying traffic between the BT and Operator Switch Connections.

If the Operator Switch is taking timing information from the BT System via a 2 Mbit/s Interconnect Links which fails (i.e. AIS is detected) then it must meet the following requirements:

A.	Switching synchronisation to an alternative 2Mbit/s Interconnect Links, if available.

- If no such synchronisation is available, entering holdover mode and keeping within the limits of holdover operation specified in section 2.2.3 of ITU-T Recommendation G.812 (Holdover Operation).

# 5.	FUNCTIONAL CHARACTERISTICS OF THE INTERFACE

Functional characteristics of the 2Mbit/s interface shall be in accordance with ITU-T Recommendations G.704 (Synchronous Frame Structures used at Primary and Secondary Hierarchical Levels) and G.706 (Frame Alignment and Cyclic Redundancy Check (CRC) Procedures Relating To Basic Frame Structures Defined In Rec. G704) with the following additions and clarification's:

## 5.1.	Signalling

If Time Slot 16 is not required for signalling information, it must not be used as a traffic carrying channel within the BT System.

Signalling across the interface is not specified in this document.

## 5.2.	Timeslot '0'

Chapter 2.3 of Rec. G.704 (Basic Frame Structure at 2048 kbit/s) applies. Bits 4-7 in time slot zero not containing the frame alignment signal should be set to "1". The use of bit 8 for the return direction shall be determined by BT at each location. On some systems bit 8 will be set to "1" in the go and return direction. On other systems when BT detects one or more errors in the frame alignment word, this bit 8, in the return TSO "not" word, will be set at a "1" state on two successive occasions; when no errors are detected bit 8 will be set to the "0" state. If possible, the Operator Switch should make the same use of this bit 8, if not it should be tolerant to the sending of bit 8 in the return direction and set it to "0" in the go direction.

## 5.3.	Alarm Indication Signal (AIS)

Under certain fault conditions AIS is used in the BT System. AIS is indicated by a continuous stream of binary 1's. When transmitted AIS is controlled by a free running 2048 kbit/s crystal oscillator (accuracy within ± 50 ppm).

The strategy for detecting the presence of AIS should be such that AIS is detectable, even in the presence of an error ratio of 1 in 1000. However, a signal with all bits except the frame alignment word in the '1' state, should not be mistaken as an AIS.

## 5.4.	Channel Time Slot Encoding

The 64 kbit/s channel time slots comprising the 2048 kbit/s stream shall carry 'A' law encoded information as defined in ITU-T Recommendation G.711 (Pulse Code Modulation (PCM) Of Voice Frequencies).

The idle channel bit pattern transmitted over the Interconnect Link shall be compliant with ITU-T Recommendation Q.522 section 2.12 (Bit Patterns Generated By The Exchange In Idle Channel Time slots).

# 6.	SAFETY AND PROTECTION

## 6.1.	Dangerous Voltages

In order to protect personnel and equipment on both sides of a Point Of Connection, it is necessary to provide protection against the transmission of excessive voltage across the interface.

Excessive voltages shall be as defined in BS 6301: 1989. For equipment which uses or generates excessive voltages the interface shall be electrically isolated from those voltages. Suitable devices are described in BS 6301: 1989.

## 6.2.	Radiation Hazards

Where radio equipment is used, arrangements shall be made to protect all personnel from levels of radiation exceeding 1 milliwatt per square centimetre.

# 7.	REFERENCES

ITU-T

G.703	Physical/Electrical Characteristics of Hierarchical Digital Exchanges

G.704	Synchronous Frame Structures used at Primary and Secondary Hierarchical Levels.

G.706	Frame Alignment and Cyclic Redundancy Check (CRC) Procedures Relating To Basic Frame Structures Defined In Rec. G704

G.711	Pulse Code Modulation (PCM) Of Voice Frequencies.

G.811	International Connections Terminating on Synchronous Network Nodes

G.812	section 2.2.3 (Holdover Operation).

G.823	The Control of Jitter and Wander Within Digital Networks Which Are Based on the 2048 KBIT/S Hierarchy

G.823	section 3.1.1 Jitter And Wander Tolerance of Digital Input Ports

G.957 Optical  Interfaces  For  Equipments  And  Systems  Relating  To  The  Synchronous  Digital  Hierarchy

0.151	Error Performance Measuring Equipment for Digital Systems At The Primary Bit Rate and Above

O.171	Timing Jitter Measuring Equipment for Digital Systems

Q.522	Section 2.12 Bit Patterns Generated By The Exchange In Idle Channel Time slots

Q.551	Transmission Characteristics of Digital Exchanges

BS6301 1989. Safety Requirements for Apparatus for Connection to British Telecommunication Networks

# 8.	GLOSSARY

mF	-	Micro Farad

ms		-	Microsecond

2Mbit/s-	2048kbit/s

AIS	-	Alarm Indications Signal

BS	-	British Standard

CTU	-	Circuit Terminating Unit.

dB	-	Decibel

DC	-	Direct Current

DDF	-	Digital Distribution Frame

ITU-T	-	International Telecommunication Union - Telecommunications

kbit/s	-	KiloBits per second

kHz	-	Kilo Hertz

Mbit/s	-	Megabits per second

SDH	- Synchronous Digital Hierarchy

ppm	-	Parts per million

UI		-	Unit Interval

# 9	HISTORY

| Issue 1 | January 1994 |  |
| --- | --- | --- |
| Issue 2 draft 1 | June 1994 |  |
| Issue 2 | July 1994 |  |
| Issue A | August 1994 |  |
| Issue B | Sept 1994 |  |
| Issue C | January 1996 |  |
| Issue D | February 1997 |  |
| Issue E | June 1998 |  |
| Issue F | January 1999 |  |
| Issue G | April 2000 | Aggregate STM 4 added for CSI in 2.2 |

END OF SPECIFICATION

BT Interoperability and Technical Regulatory Services

PP 2D27, Angel Centre

403 St John Street

London EC1V 4PL