GENERIC

TRANSMISSION

INTERFACE SPECIFICATION

I/C Specification 0100

GENERIC TRANSMISSION INTERFACE SPECIFICATION

CONTENTS:

1. GENERAL	3

2. RESPONSIBILITIES	3

3. RELEVANT PARAMETERS	4

4. OVERALL TRANSMISSION LOSS	4

4.1 Loudness Rating (LR)	4

5. APPORTIONMENT OF OVERALL LOUDNESS RATING (OLR)	4

5.1 End to End Requirements	4

5.2 GSM/PCN Loudness Control	5

5.3 RLR And Receive GSM Volume Control	5

6. TRANSMISSION TIME	5

6.1 Transmission Delays Within The Fixed Network - Without echo control	5

6.2 Preferred Apportionment Of Transmission delay	6

6.3 Calls that involve an International Gateway	6

6.4 Maximum delay under route failure conditions.	6

6.5 Transmission Delays Within The Fixed Network - With echo control	6

6.6 Transmission Delays Involving Digital Mobile/Wireless Access	7

6.6.1 Transmission Delays Between Digital Mobile/wireless access and the Fixed Network - With echo control	7

6.6.2 Transmission Delays Within Digital Mobile/wireless access Networks - With echo control.	7

6.7 Transmission Delays On International Calls - With echo control.	7

7. ECHO LOSS	7

7.1 GSM/PCN Echo Loss	8

7.2 Echo Control - International Conformance	8

7.3 Echo Control - GSM	8

8. STABILITY LOSS	8

9. QUANTISING DISTORTION	9

9.1 Fixed Network Limits	9

9.2 Mobile Network limits	9

10. CODING STANDARDS	9

11. NOISE	10

12. ATTENUATION DISTORTION	10

13. GROUP DELAY DISTORTION	10

13.1 Sidetone Masking Rating - Normal Telephony Customer to BT PSTN.	10

13.2 Sidetone Masking Rating BT PSTN to GSM/PCN	11

14. ERROR PERFORMANCE	11

15. NON SPEECH LEVELS	11

16. GSM/PCN HEADSET AND HANDSET SENSITIVITY/FREQUENCY CHARACTERISTICS	11

17. REFERENCES	12

18. HISTORY	13

# GENERAL

This Specification sets out the transmission requirements and objectives to be met by BT and the Operator for the interconnection of the BT and Operator Systems. The requirements are based on the relevant International Telecommunications Union (ITU), European Telecommunications Standards Institute (ETSI) and Public Network Operator - Interest Group (PNO-IG) Recommendations.

References to Global Systems Mobile / Personal Communications Networks (GSM/PCN) are based upon ETSI/ GSM recommendation. 3.5 phase 1. This recommendation is currently under review and will be updated to ETSI 300 540 GSM Phase 2.

End to end requirements involve the performance of Customer equipment which is outside BT and the Operators controls.

Customer networks as referred to throughout this document typically consist of at least one item of Customer Premises Equipment (CPE) situated beyond the Network Terminating Point (NTP).

Where this Specification sets out matters that the Parties endeavour to agree, and agreement is not reached, such matters shall be disputes.

# RESPONSIBILITIES

The responsibility for overall transmission quality is held by the Party selected by the Customer to carry the Call. If the Call is an indirect access Call then the indirect access Operator is responsible for the end to end quality of the Call.

If the overall transmission quality for a Call cannot be met because either Party is unable to comply with the reasonable requests of the other Party, then neither Party will be obliged to convey the Call.

# RELEVANT PARAMETERS

Overall performance is affected by the following parameters:

Overall loss

Transmission Time (Absolute delay and propagation delay)

Echo and Stability

Quantising distortion

Coding standards

Attenuation distortion

Group delay distortion

Sidetone loss

Crosstalk

Errors

Jitter and Wander

BT and the Operator shall endeavour to achieve the requirements and objectives for overall loss, delay, echo loss, quantising distortion, and coding standards given in sections 4 to 10 inclusive. BT and the Operator shall each plan in accordance with the guidance given for the remaining parameters listed above.

It should be noted that for some parameters the CPE may have a significant effect on the end to end performance.

# OVERALL TRANSMISSION LOSS

## Loudness Rating (LR)

The limits for overall loss expressed in terms of Loudness Ratings (LR) are defined in ITU-T Recommendation P. 76 (Blue book November 1989).

# APPORTIONMENT OF OVERALL LOUDNESS RATING (OLR)

## End to End Requirements

BT and the Operator shall endeavour to:

a) Provide connections which fall within the ITU-T G111 recommended OLR range of 8dB to 20dB for all analogue or mixed analogue/digital routings. For all digital routings the mean value for OLR shall be in the range 8dB to 12dB.

b) avoid connections which exceed the ITU-T Recommended limiting OLR value of 29dB

c) minimise the range of different Transmission levels experienced by any one Customer.

## GSM/PCN Loudness Control

Cellular network operators may utilise digital level control in the mobile switched network to control Send Loudness Rating (SLR) and Receive Loudness Rating (RLR). values.

.

## RLR And Receive GSM Volume Control

The use of any Customer controlled receive volume control shall not decrease (i.e. make more sensitive) the RLR, by more than 10 dB for planning purposes.

# TRANSMISSION TIME

## Transmission Delays Within The Fixed Network - Without echo control

For UK Calls not employing echo control, the NTP - NTP one way delay shall be less than 13 ms for at least 95% (See) of Calls. Assuming that Customer network delays at each end of the Call do not exceed 5 ms, then the total end to end delay shall be less than 23 ms.

For the small proportion of Calls that exceed the 13ms limit, an absolute limit of 25ms excluding Customer networks shall be observed. Previous investigations have identified that customers may find such Calls to be unacceptable since Customer network delays could increase the end to end delay to over 30ms.

## Preferred Apportionment Of Transmission delay

The apportionment principles presented in Figure 1 ensure that BT or Operator Customers experience acceptable levels of transmission delay. Any reapportionment of the segment boundaries (Collection, Transport and Delivery) shall be subject to joint agreement.

## Calls that involve an International Gateway

For International Calls the one way delay from the NTP to the International Switching Centre (ISC) shall be no greater than 7 ms for 95% of Calls, with an absolute limit of 12 ms.

## Maximum delay under route failure conditions.

In situations where Calls have to be re-routed around failed sections of the BT or Operator Network, it is acceptable for the proportion of Calls meeting recommended delay limits to fall below 95%, however the absolute limits given above shall not be exceeded.

## Transmission Delays Within The Fixed Network - With echo control

Where echo control is provided over the fixed network, the one-way delay limit for at least 95% of national Calls shall be less than 125 ms. No Calls shall exceed the absolute one-way delay limit of 150 ms.

## Transmission Delays Involving Digital Mobile/Wireless Access

### Transmission Delays Between Digital Mobile/wireless access and the Fixed Network - With echo control

The one way delay limit for at least 95% of Calls between mobiles on digital networks or wireless access networks and the fixed network shall be less than 125 ms. No Calls shall exceed the absolute one-way delay limit of 150 ms.

### Transmission Delays Within Digital Mobile/wireless access Networks - With echo control.

The one way delay limit for at least 95% of Calls between mobiles on digital networks or wireless access networks within the UK shall be less than 215 ms. No connection shall exceed 230 ms one way delay.

## Transmission Delays On International Calls - With echo control.

The maximum one way delay for an International Call using echo control will depend upon the nature of the destination network e.g. fixed, digital mobile or wireless access and the transmission media e.g. satellite or cable.

International Calls using echo control should not exceed 400 ms, it is however recognised that under cable circuit failure/congestion conditions, satellite backup circuits may not meet this requirement.

# ECHO LOSS

BT and the Operators shall seek to meet the design objective for echo loss (as defined in ITU-T Recommendation G122) presented at the Switch Connection with equal relative levels for both directions of transmission of 20dB, with no connections being less than 15dB for practical implementation.

CPE connected via a 2-wire interface can have significant effect on echo losses. For planning purposes BT and the Operator shall assume that CPE will present an impedance of 600 ohms or the network shown in fig 5 of BS6305 at the 2 wire NTP.

CPE and Customer networks are likely to determine the largest part of echo losses in the case of 4-wire connection to the BT and the Operator Systems. For planning purposes BT and the Operator shall assume that the CPE meets the 20dB echo loss objectives given in the Oftel Network Code of Practice (for the Design of Private Telecommunications Networks) - NCOP.

## GSM/PCN Echo Loss

The echo loss under operational conditions for a GSM/PCN based system shall be at least 46 dB referred to the Switch Connection with any Customer volume control set to its maximum output (i.e. loudest) position. ETSI/GSM 03.50/1 Section 3.4.1 provides further information related to echo loss. ITU-T Recommendation G165 provides guidance for the performance of echo control devices when switched into a connection. ITU-T Recommendations G151 and G473 refer.

## Echo Control - International Conformance

Echo control devices for international connections to and from BT shall conform to ITU-T. Rec.G.165 on echo cancellers.

## Echo Control - GSM

The GSM/PCN system shall provide echo protection as stated in ETSI/GSM 03.50/1 Section 3.4.2.

# STABILITY LOSS

For International Calls and National Calls stability loss considerations shall be satisfied by the following requirement.

The loss presented at the Switch Connection with equal relative levels for both directions of transmission shall not be less than 6dB at any frequency up to 4kHz. Account shall be taken of all conditions presented at the Switch Connection under normal operating conditions and any customer volume controls should be at maximum output.

CPE connected via a 2-wire interface may have a significant effect on stability loss. For planning purposes BT and the Operator shall assume the CPE may present either open circuit or short circuit conditions at a 2-wire NTP.

CPE and Customer networks are likely to largely determine the stability losses in the case of 4-wire connection to the BT and the Operator Systems. For planning purposes BT and the Operator shall assume that the CPE meets the 6dB stability loss requirement given in the NCOP.

It is recognised by the Parties that stability losses of less than 6 dB could cause oscillation.

# QUANTISING DISTORTION

## Fixed Network Limits

In order to meet international and UK requirements the following limits for quantising distortion units (qdu) shall be met:

A1) Max end to end International				14 qdu

B1) Public network collection	(access network)		5 qdu

C1) National transport/Trunk Network			0 qdu

D1) Public network delivery (access network)		5 qdu

## Mobile Network limits

D2) Max end to end International				18 qdu

A2) Collection	(access	network)				7 qdu

B2) National transport/Trunk Network			0 qdu

C2) Delivery (access network)				7 qdu

For planning purposes BT and the Operator shall assume that Customer networks do not introduce more that 2 qdu.

# CODING STANDARDS

At a digital interface it is a requirement that analogue information shall be encoded using the 8bit, A-law characteristic in accordance with ITU-T Recommendation G711 such that a 64kbit/s time slot at the Switch Connection can be decoded using a 8 bit, A-law decoder.

# NOISE

The limits for single tone or narrow band noise shall be more stringent than the limits for wideband noise to avoid Customer annoyance. As a general rule, the power in any individual tone should be 10dB less than the psophemetric noise power in the circuit (ITU-T Recommendation P11).

The following ITU-T recommendations shall be complied with to give appropriate limits.

Digital Exchanges - Recommendations Q.551 and Q.554

PCM line systems - Recommendation G.712

GSM/PCN systems - ETSI/GSM 3.50/1 section 3.2

The limits in Recommendation G.123 are of particular importance as they control the level of noise on International Calls.

# ATTENUATION DISTORTION

In order to adequately control attenuation distortion each component of the connection shall have a suitable distortion limit. The following ITU-T Recommendations apply.

Digital Exchanges - Recommendations Q.551 and Q.554

Digital Line Systems - Recommendation G.712

# GROUP DELAY DISTORTION

The following ITU-T Recommendations give suitable limits for the group delay distortion introduced by line transmission systems and coding processes in digital exchanges:

Digital Exchanges - Recommendations Q.551 and Q.554

Digital Line Systems - Recommendation G.712

## Sidetone Masking Rating - Normal Telephony Customer to BT 			PSTN.

The Sidetone Masking Rating (STMR) for telephony CPE connected to a BT Network Terminating Point shall nominally be taken as 7 dB for planning purposes.

## Sidetone Masking Rating BT PSTN to GSM/PCN

The Sidetone Masking Rating (STMR) for telephony CPE allowing users to gain access through the GSM system shall be nominally taken as outlined in ETSI/GSM 03.50/1 for STMR and LSTR.

# ERROR PERFORMANCE

Error performance of digital networks is of key importance as it determines the end to end performance of both end to end digital services and analogue services supported by the BT and Operator Systems.

The allocation principles of Recommendation G. 821 shall be used when determining the error for individual transmission systems.

# NON SPEECH LEVELS

The use of any non-speech signals within an established Call, or during the phase of Call set-up or clear down, for such purposes as signalling (e.g. DTMF) shall comply with the principles set in Section 4 of BS6305 : 1992, that have been based upon a 0 dBr interface.

Interfaces that are not taken as a 0 dBr point shall be planned as if they were a 0 dBr for purposes of realising interconnection between the BT System and a PCN/ GSM System.

# GSM/PCN HEADSET AND HANDSET SENSITIVITY/FREQUENCY CHARACTERISTICS

The GSM/PCN system shall provide a performance in accordance with Section 3.8 of ETSI/GSM Recommendation 03.50/1.

# REFERENCES

ITU-T recommendations

G.111 Loudness Ratings in an International Connection

G.122 Influence Of National Systems On Stability, Talker Echo and Listener Echo In International Connections

G.123 Circuit Noise In National Circuits

G.151 General performance objectives applicable to all modern International and National extension circuits.

G165 Echo cancellers

G473 Interconnect of a Maritime Mobile satellite system with the International automatic switched telephone service: Transmission aspects

G.711 Pulse code modulation (PCM) of voice frequencies.

G.712 Performance characteristics of PCM channels between 4-wire interfaces at voice frequencies.

G.821 Error performance of an International digital connection forming part of an integrated services digital network.

P.11 Effect Of Transmission Impairments

P.16 Subjective effects of direct crosstalk; Thresholds of audibility and intelligibility

P.76 Determination of Loudness rating; Fundamental principles

Q.551 Transmission characteristics of digital exchanges.

Q.554 Transmission characteristics at digital interfaces of a digital Exchange.

BS Specifications

BS 6305

NCOP - OFTEL Network Code Of Practice for the design of Private Telecommunications Networks

ETSI Specifications

ETSI/GSM Recommendation 03.50

# HISTORY

Issue 1.0	January 1994

Issue 1.1	February 1994

Issue 2 Draft	1 June 1994

Issue 2	July 1994

Issue 2.1	7 July 1994

Issue A	August 1994

Issue B	August 1994

Issue C	January 1996

Issue D	June 1998

END OF SPECIFICATION

BT Interconnect Standards

PP 09A14, Delta Point

35 Wellesley Road

Croydon

CR9 2YZ

The  95% limits in , , , ,  &   shall be implemented such that the possibility of individual Customers always encountering unacceptable performance  is minimised.