ANNEX A

PLANNING AND OPErations

INDEX

A1	Definitions

A2	General

A3	Network Information

A4	Location of Point of Handover

A5	ISH Link Architecture

A6	Capacity Profiles and Advance Capacity Orders

A7	Capacity Orders and Testing

A8	Capacity Order Timescales

A9	Technical Review Meetings

A10	Link Testing

A11	Transmission and Synchronisation

A12	Performance Standards

A13	Operations

A14	Restoration

Appendices

A	Information

B	Commercial

1.	DEFINITIONS

1.1	In this Annex and the Appendices, a reference to a paragraph or Appendix, unless stated otherwise, is to a paragraph or appendix of this Annex. Words, abbreviations and expressions have the meanings given in Annex D.

2.	GENERAL

2.1	This Annex details the planning and operational principles for the provision and re-arrangement of ISH Links and Cross Connection and the provision of CSH Links and associated Backhaul Capacity where appropriate, and provides the technical basis for the Specifications and for the Manuals. The Manuals provide procedures for the provision and testing of Capacity (Provisioning Manual) and operations and maintenance (Operations and Maintenance Manual).

2.2	The details of the ISH Links and CSH Links, and associated Backhaul Capacity where appropriate, and the plans for future developments shall be recorded in the Technical Master Plan. The Customer Service Plan shall contain details of the points of contact within the Operator’s and BT’s organisation. The Parties shall endeavour to keep these documents up to date.

3.	NETWORK INFORMATION

3.1	Insofar as not previously provided, BT shall provide to the Operator the information listed in Appendix A as soon as reasonably practicable and in any event not more than 20 Working Days from the date of this Agreement.

3.2	In addition to the requirements set out in paragraph 5 of the main body of this Agreement, BT will provide the Operator with information on any proposed introduction, closure, replacement or modification to any Cable System referred to in paragraph 3.1, including changes to equipment where such changes affect the equipment that the Operator will be required to use, as soon as practicable and, in any event, not less than seven months prior to the implementation.

4.	LOCATION OF POINTS OF HANDOVER

4.1	Points of Handover may be constructed at the following locations:

4.1.1	Cable Stations where the Cable Station supports the relevant Cable System;

4.1.2	ITCs where the ITC has transmission capacity directly connected to the relevant Cable System.

4.1.3	in the case of any CSH Link, the relevant Operator building.

5.	ISH LINK ARCHITECTURE

ISH Links and CSH Links shall be transmission systems using either PDH or SDH technology, where the interfaces are set out in the Cable Specification and, in the case of SDH, additionally in the Generic SDH Interface Specification.

5.1	ISH Links using PDH

(a)	Equipment

The manufacturer and type of equipment used to terminate the Line System shall be the same at each end of the fibre and be one of those agreed and listed in the Provisioning Manual.

(b)	Procedures

Operational procedures for the provision of PDH ISH Links are described in the Provisioning Manual and their maintenance is described in the Operations and Maintenance Manual.

(c)	Path Protection

Where the Operator requests Path Protection using PDH, as set out in Schedule 01, the manufacturer and type of equipment used to effect the Path Protection shall be the same at each end of the fibre and be one of those agreed and listed in the Provisioning Manual as suitable for supporting Path Protection.

5.2	ISH Links using SDH

(a)	Configuration

ISH Links using SDH will deliver the equivalent functionality to an ISH Link using PDH technology.

For the avoidance of doubt, if an ISH Link uses SDH technology there shall, unless the Operator has requested Path Protection, be one single physical path between the ADMs.

(b)	Equipment

The manufacturer and type of equipment that BT shall use to terminate the SDH Line System shall be those listed in the Provisioning Manual. The Operator may use any equipment that conforms to the Generic SDH Interface Specification.

(c)	Management

The Operator shall exercise management control of the ISH Link, and associated Backhaul Capacity where appropriate unless the Parties agree otherwise.

(d)	Testing and Operational Procedures

If the Operator chooses to use equipment that has not been validated to ensure that it interworks with the equipment that BT uses, the Parties shall test the equipment to ensure that the performance standards are met.  The Operator shall meet BT’s reasonable costs for such testing.  Unless otherwise agreed, such testing shall be completed before an order for such an ISH Link is placed. If the Parties agree to carry out the testing after an order is received that order shall be subject to adjustment of the delivery date to include such tests. Operational procedures for the provision of SDH ISH Links are described in the Provisioning Manual and their maintenance is described in the Operations and Maintenance Manual.

(e)	Path Protection

If the Operator requests Path Protection, all Virtual Container (VC) paths shall terminate on a single ADM at the ITC or Cable Station (as appropriate). This configuration shall be used where Virtual Container (VC) Path Protection is deployed in the SDH equipment terminating the Virtual Container (VC) paths.

6.	CAPACITY PROFILES AND ADVANCE CAPACITY ORDERS

6.1	Before placing Capacity Orders, the Operator shall (subject to paragraph 6.2 below) supply BT with Capacity Profiles in relation to Capacity Provision and Capacity Re-arrangements for all existing and proposed Cable Systems for each Point of Handover.

6.2	Capacity Profiles for Capacity Provision and Capacity Re-arrangements shall be on a rolling two year basis and provided on forms as described in the Provisioning Manual. The first six months of each Capacity Profile shall form an Advance Capacity Order pursuant to paragraph 7.

6.3	Each Capacity Profile shall be presented to BT at six month intervals, not less than 15 Working Days before the relevant Quarter Day.

6.4	Where an ACO exceeds the Capacity Profile for the period of 6 months in question to the extent that new digital Cross Connect equipment is required, BT shall advise the Operator within 20 Working Days of receipt of the Capacity Profile.

7.	CAPACITY ORDERS AND TESTING

7.1	Capacity Ordering

7.1.1	In an ACO Period the Operator may place Capacity Orders up to the amount of the aggregate Capacity (including the provide element of Capacity Re-arrangements) shown in the relevant Advance Capacity Orders for each Cable System for each Point of Handover. Within this limit BT shall provide such Capacity that it is to provide within the time scales laid down in paragraph 8.

7.1.2	In an ACO Period the Operator shall place Capacity Orders for a minimum of 80 per cent. of the aggregate Capacity (including the provide element of Capacity Re-arrangements) specified in the relevant Advance Capacity Orders for each Cable System for each Point of Handover. Failure by the Operator to place such Capacity Orders shall result in pre-payment charges being paid by the Operator in accordance with paragraph 1 of Appendix B. Pre-payment charges arising shall be accepted as pre-payment for future Capacity Orders in respect of Capacity for the Cable System at each Point of Handover placed within 12 months of the end of the relevant ACO Period (thereby being credited against any charges payable by the Operator in respect of such Capacity Order(s)).

7.1.3	If no Advance Capacity Order is provided or Capacity Orders are placed by the Operator in excess of the relevant limit in paragraph 7.1.1:

(1)	BT shall not be obliged to meet the time scales set out in paragraph 8 for that element of the Capacity Order in excess of the relevant limit; and

(2)	the time scales shall be agreed in writing for the provision of excess Capacity and the Ready for Test Dates shall be not later than 6 months after the date of receipt of a relevant Capacity Order unless a new Cross Connection is required involving new digital cross connect equipment, in which case 12 months shall apply; and

(3)	if the Capacity is in excess of the relevant limit BT may raise charges to cover any additional costs that it may incur arising directly from the provision of Capacity outside the normal planning process. BT will not incur such additional costs or provide Capacity outside the Advance Capacity Order process without the prior written approval of the Operator.

7.1.4	Each Capacity Order will be in the form as advised by BT from time to time and shall include all the information required by that form.

8.	CAPACITY ORDER TIMESCALES

## 8.1  		Timescales for Provision of Capacity for In Span Handover

The timescales for the provision of Capacity are subject to the provisions set out in Annex C and paragraph 7.1.3.  In all cases, a Cross Connection will be provided in respect of any Submarine Cable Capacity following receipt of a separate order to activate such Capacity.  The timescales are:-

8.1.1  	Capacity Provision passing through a new Point of Handover using PDH or SDH:-

6 months from the date of receipt of the relevant Capacity Order to Ready for Test Date, unless a Cross Connection is required and this involves installing new Cross Connection equipment as a result of the relevant Advance Capacity Order exceeding the Capacity Profile for the period of 6 months in question, in which case 12 months shall apply.

8.1.2  	Capacity Provision or Capacity Re-arrangement requiring a change of technology from PDH to SDH or SDH to PDH at an existing Point of Handover:-

6 months from the date of receipt of the relevant Capacity Order to Ready for Test Date, unless a Cross Connection is required and this involves installing new Cross Connection equipment as a result of the relevant Advance Capacity Order exceeding the Capacity Profile for the period of 6 months in question, in which case 12 months shall apply.

8.1.3	ISH Link Augmentation, together with associated Backhaul Capacity where appropriate, using the same technology in units of 2 Mbit/s, in units of 34 Mbit/s or in units of 140 Mbit/s where an STM-4 Multiplexor has previously been provided:-

25 Working Days from the date of receipt of the relevant Capacity Order to Ready for Service Date, this period not including the activation of capacity in the Cable System or the provision of Cross Connection.

8.1.4	ISH Link Augmentation, together with associated Backhaul Capacity where appropriate, using the same technology in units of 140 Mbit/s where a new STM-1 or STM-4 Multiplexor is required:-

6 months from the date of receipt of the relevant Capacity Order to Ready for Test Date, unless a Cross Connection is required and this involves installing new Cross Connection equipment as a result of the relevant Advance Capacity Order exceeding the Capacity Profile for the period of 6 months in question, in which case 12 months shall apply.

8.1.5	Capacity Re-arrangements involving a change of Cable System accessed from the same Cable Station:-

25 Working Days following the date of receipt of the relevant Capacity Order to the Ready for Service Date, this period not including the activation of capacity in the Cable System and provision of Cross Connection.

8.1.6	Cross Connection – 35 Working Days following the date of receipt of the relevant order under Schedule 02 to Ready for Service Date.  Such period may run concurrently with any other order placed under this paragraph 8.1.

8.1.7	Path Protection – on a date such as the Parties shall agree.

8.1.8	The above provisions are summarised in the Table below.

| Order type | Lead Time |
| --- | --- |
| New Point of Handover | 6 months. |
| Re-arrangement requiring change from PDH to SDH or vice versa | 6 months |
| Provision or Re-arrangement requiring new Cross Connect equipment as a result of ACO exceeding Capacity Profile | 12 months |
| Re-arrangement involving change of Cable System, but not Cable Station | 25 Working Days, not including Cross Connection. |
| VC3 to existing Points of Handover | 6 months |
| 140 Mbit/s/VC4 to existing Point of Handover | 6 months |
| Additional E1(2 Mbit/s) to existing Point of Handover | 25 Working Days, not including Cross Connection. |
| Cross Connection | 35 Working Days |

8.2	Timescales for Provision of Capacity for Customer Sited Handover

The timescales for the provision of Capacity are subject to the provision set out in Annex C and paragraph 7.1.3.

8.2.1	Capacity Provision for 2 Mbit/s CSH Links and appropriate Backhaul Capacity:-

60 Working days from the date of receipt of the relevant Capacity Order to Ready for Service Date, including activation of capacity in the Cable System and the provision of Cross Connection.

8.2.2	Capacity Provision for high bandwidth CSH Links and appropriate Backhaul Capacity:-

| Order type | Lead time |
| --- | --- |
| First VC4 CSH Link to Operator’s building | Up to 6 months |
| First VC3 CSH Link to Operator’s building | Up to 6 months |
| Provision or re-arrangement requiring new Digital Cross Connect equipment as a result of Advance Capacity Order exceeding Capacity Profile | 12 months |
| Re-arrangement involving change of Cable System, but not Cable Station | 35 Working Days, including Cross Connection. |
| Subsequent VC4 CSH Links within Operator’s Advanced Capacity Order to an existing Point of Handover | 35 Working Days, including Cross Connection. |
| Subsequent VC3 CSH Links within Operator’s Advanced Capacity Order to an existing Point of Handover | 35 Working Days, including Cross Connection. |
| Digital Cross Connection of sub-sea cable capacity to an existing VC3 or VC4 CSH Link and appropriate Backhaul Capacity | 35 Working Days |

8.2.3	Capacity Rearrangements of the types set out in the Carrier Price List:-

On a date such as the Parties shall agree.

8.3	Cancellation of Order

At any time prior to the Ready for Test Date the Operator may by written notice to BT cancel a Capacity Order it placed. If a Capacity Order is cancelled more than 10 Working Days after placement of the Capacity Order, the Operator shall pay a cancellation charge as set out in paragraph 2 of Appendix B.

8.4	Delivery

If BT fails to fulfil any order within the timescale set out in paragraph 8.1 or 8.2 (or such later date as may be specified in the Operator’s order) and as a direct result the Operator is prevented from accessing the relevant Submarine Cable Capacity and BT does not provide or cause to be provided alternative submarine cable capacity and terrestrial facilities, BT shall pay the Operator liquidated damages calculated in accordance with paragraph 4 of Appendix B.

8.5	Removal of Capacity

8.5.1	Subject to the relevant Schedules, if the Operator requires the removal of Capacity it paid for (or for which payment is due by the Operator under this Agreement), an order identifying the Capacity and the date from which it is no longer required (a “removal order”) may be placed by the Operator on BT.  BT will remove the Capacity not later than 30 Working Days from the date of receipt of the removal order and a removal certificate shall be issued to the Operator on completion of the removal work.

8.5.2	For the avoidance of doubt, if payment for Capacity removed pursuant to this paragraph 8 has not been made at the time of such removal such payment shall remain due and payable.  Rental payments shall be payable by the Operator up to the earlier of the date of completion of the removal work or 30 Working Days date of receipt of the removal order.

8.6	Capacity Re-arrangement

The Operator may request in writing Capacity Re-arrangement, and BT shall carry out the necessary work in accordance with the time scales set out in paragraph 8.1 and 8.2. Capacity Orders, including any placed in the last 10 Working Days of an ACO Period which bring the total Capacity Orders up to 80% of the Capacity specified in the ACO, and subsequently cancelled within 10 Working Days of placing those orders shall not be included in the amount of Capacity ordered in the relevant ACO period. BT shall issue a certificate to the Operator on completion of work. The Purchaser may submit requests for Capacity Re-arrangement in writing to the Supplier at any time after the earlier of Ready For Service Date or one month after Ready For Test Date.  On completion of a Capacity Re-arrangement the original Capacity is deemed terminated and replaced by the rearranged Capacity.

9.	TECHNICAL REVIEW MEETINGS

9.1	The first Technical Review Meeting shall occur at either Party's written request not later than six months from the receipt of the first Capacity Profile. Subsequent Technical Review Meetings shall occur at no greater than six monthly intervals thereafter, not less than 10 Working Days prior to the start of the relevant six month period next following.

9.2	At Technical Review Meetings the Capacity Profile referred to in paragraph 6 shall be discussed.  This meeting may be used to co-ordinate the Parties activities in respect of this Agreement and in respect of administration of the relevant Cable Systems.

10.	LINK TESTING

10.1	Testing Principles

(1)	BT shall give the Operator advance notice that Capacity is Ready for Testing.  Where practicable, such notice shall be at least 10 Working Days.  Such notice shall be in the form set out in the Provisioning Manual.  BT and the Operator shall commence testing the Line System not later than 25 Working Days after the relevant scheduled Ready for Test Date.  The Parties shall co-operate to ensure that testing takes place on dates and timescales agreeable to both Parties.  The Parties shall use reasonable endeavours to complete testing in the shortest appropriate time.

(2)	As soon after the Ready for Service Date as is practical, both Parties shall certify that the Capacity is satisfactory by signing the certificate set out in the Operations and Maintenance Manual.

(3)	If for any reason the Operator is unable to proceed jointly with BT in testing of the Line System the Operator shall notify BT in writing as soon as possible and where such notice is less than 5 Working Days before the scheduled Ready for Test Date, the provisions set out in paragraph 3 of Appendix B of this Annex shall apply.  Testing shall then be re-scheduled by agreement and commenced on such re-scheduled date.  In the event of testing failing the Parties shall use reasonable endeavours to ensure testing is recommenced and concluded as quickly as possible.

(4)	The Parties shall agree the required test facilities.  The Parties shall make such test facilities available from the date required by the other Party.

10.2	BT shall deliver Capacity on ISH Links, CSH Links and Backhaul Capacity tested and Ready for Service.  BT shall conduct such tests as it deems appropriate to confirm satisfactory operation of the ISH Links and the CSH Links.  The Operator shall be responsible for all further testing including testing of the Cable System.

10.3	In the event that the Parties fail to agree any test procedure and/or time scale, then either Party may notify the other in writing of a Dispute.

10.4	If the Operator requires BT’s assistance in testing beyond that contemplated in paragraph 10.1 and 10.2, the Parties shall agree the scope timescales and charges for such assistance.

11.	TRANSMISSION

11.1	ISH Links

ISH Links shall employ digital technology operating in accordance with, if appropriate, the Generic SDH Interface Specification.  The Operator shall provide synchronisation for the equipment associated with the ISH Link.

11.2	CSH Links

CSH Links shall employ digital technology at a circuit data rate of 2048  Kbit/s, VC3 or VC4 and terminate at the Operator building on a pair of unbalanced 75 ohm BNC sockets, where one socket is for each direction of transmission.  The electrical interface conforms to ITU-T recommendation G.703 and is neither timed nor structured.

12.	PERFORMANCE STANDARDS

12.1	General

12.1.1	The Parties shall co-operate to maintain the overall quality of the ISH Links, and associated Backhaul Capacity where appropriate, and adopt the general principles on standards, techniques and methodology for the achievement of quality in telecommunications networks and services contained in the relevant ITU-T recommendations.

12.1.2	The Parties shall agree operational management strategies to protect service quality.

12.2	Service Performance

12.2.1	BT shall provide the Cross Connection, Backhaul Capacity and the ISH Link (excluding the Line System) during those periods of time and at the same quality of service as it provides such service for its own use.

12.2.2	BT shall provide its part of the Line System such that the performance of its part meets or exceeds that set out in the following ITU-T recommendations: G.823, G.825, G.958, M.2100 and M.2101.

12.2.3	BT shall provide CSH Links and Backhaul Capacity during those periods of time and at the same quality of service as it provides similar capacity on the UK land based portion of its international private leased circuits to its Customers.

12.3	Information on Service Performance

Each Party ("the first Party") shall provide to the other, as the other Party may reasonably require, information on service performance of the first Party's System from time to time for the purpose of the other Party verifying the performance by the first Party of its obligations under this Agreement, including without limitation the performance standards referred to in this paragraph 12, routine maintenance time scales and fault repair times as set out in the Operations and Maintenance Manual.

13.	OPERATIONS

13.1	General

13.1.1	Each Party shall be responsible for the safety and operation of its own System.

13.1.2	The Operations and Maintenance Manual details the responsibilities, methods and procedures for the operation and maintenance of the interconnection of the Systems. Details of contacts are given in the Customer Service Plan.

13.2	Fault Identification and Reporting

13.2.1	The Operator will advise its Customers to report all faults to its fault reporting centre.

13.2.2	If an Operator Customer reports a fault to the BT fault reporting centre, BT will inform the Customer of the correct number to which to report the fault.

13.2.3	If a major fault occurs which affects both Parties' Systems, initial responsibility for identifying the fault rests with the Operator.

13.2.4	If a Party identifies a fault occurring in its System which may have an adverse effect on the other Party's System, the first Party shall promptly inform the other Party of the actions being taken to resolve the problem.

13.2.5	Each Party shall bear the costs of its own fault reporting centre.

13.3	Fault Repair

13.3.1	Where identified faults occur in the BT System that result in degradation or failure of any services provided under this Agreement for access to the Submarine Cable Capacity, BT shall investigate the fault and either instigate repair or restore service by other means.  BT shall repair the fault or restore service such that the total percentage of all service-affecting faults resulting from the particular category of event set out in the Table below, occurring in any three month period starting on a Quarter Day, is achieved in the times specified:

| PRODUCT | EVENT | TIME | PERCENTAGE (Note 1) |
| --- | --- | --- | --- |
| Schedule 01 ADM and link to Cross Connection Optical Fibre from footway box to ADM (Note 2) | Equipment failure Broken cable | 2 hours 8 hours | 80 90 |
| Schedule 02 Cross Connection | Equipment failure | 1 hour | 90 |
| Schedule 03 Backhaul | Equipment failure restored at 140 Mbit/sec | 1 hour | 90 |
| Schedule 04 CSH Link | Link failure | 4 hours | 90 |
| Schedule 05 Backhaul | Equipment failure restored at 140 Mbit/sec | 1 hour | 90 |

Note 1:	This percentage is measured as a proportion of all such faults affecting BT, the Operator and third parties.

Note 2:	If Path Protection used, time measured from notification of fault by the Operator.

13.3.2	Where a relevant fault occurs, the Operator shall be released from payment of the rental charge for the services which provide access to the relevant Submarine Cable Capacity until such time as the relevant fault is repaired or service is restored by other means.  Where such period is less than a day, the period of release shall be rounded up to a whole day.  The Operator shall provide BT with information regarding the number of service affecting faults, the number of days affected with the dates and details of the Capacity affected.  Subject to verification, BT shall reduce the next relevant quarterly rental charges to take account of such release.

13.3.3	Where any fault occurs that affects Capacity, BT shall deploy its resources to repair such fault in such a way that (when taken together with other obligation to its Customers and to third parties) there is no undue discrimination against the Operator.

13.4	Planned Maintenance/Works

13.4.1	Each Party shall provide not less than 17 Working Days notice of any planned maintenance which may affect the other's System. Each Party shall use its reasonable endeavours to minimise disruption and where possible provide alternative routing at no charge to the other Party for a reasonable period of time with respect to the cause of the disruption.

14.	CABLE SYSTEM RESTORATION

14.1	Restoration of Submarine Cable Capacity (and any charges therefor) is subject to the terms and conditions set out in the contract by which use of such capacity is acquired.  The charge for alternative Backhaul Capacity is also subject to the contract by which use of such Capacity is acquired and not this Agreement, BT making the appropriate arrangements with the owner(s) of the Cable System for payment in respect of alternative Backhaul Capacity as and when provided.

14.2	The alternative Backhaul Capacity shall be made available at an ITC in London or Madley where the Point of Handover is outside an ITC.  Where the Point of Handover is outside a Cable Station, it will be the Operator’s responsibility to make all appropriate alternative arrangements to ensure continuity of service from and between the Operator System and the appropriate ITC.  Where a CSH Link is used, restored Capacity will be delivered by BT to the relevant Point of Handover.

APPENDIX A

INFORMATION

1.	Name, address, grid reference of each ITC and Cable Station existing at the date of this Agreement.

2.	Identity of each Cable System existing at the date of this Agreement that may be accessed from a Cable Station and/or ITC.

APPENDIX B

COMMERCIAL

1.	PRE-PAYMENT CHARGES

1.1	If the Operator in any ACO Period, places Capacity Orders on BT for less than the minimum Capacity specified pursuant to paragraph 7.1.2 of Annex A, then the Operator shall pay to BT not later than 30 Working Days of the end of such ACO Period the prepayment charges calculated in accordance with paragraph 1.2 of this Appendix.

1.2	Prepayment charges are calculated in accordance with the following formula:

A	=	((80% x B) - C) x D

Where:

A	is the prepayment charge payable;

B	is the aggregate Capacity Profile (by number of units 2Mbit/s Capacity) for the relevant ACO Period in the relevant Advance Capacity Order;

C	the Capacity (by number of units of 2Mbit/s Capacity) ordered during such ACO Period not including cancellations of Capacity Orders made during or after the relevant ACO Period;

D	the relevant connection charges for the Capacity forecasted but not ordered.

2.	ORDER CANCELLATION

The charge payable pursuant to paragraph 8.3 of Annex A shall be 50% of the applicable connection charge specified from time to time in the Carrier Price List.

3.	CHARGES IN RESPECT OF LATE NOTIFICATION OF TESTING

If the notice by the Operator, pursuant to paragraph 10.1(3) of Annex A is given less than five Working Days before the scheduled Ready for Test Date, the Operator shall pay, on demand, a charge of 20% of the relevant connection charge, or charges, specified from time to time in the Carrier Price List and referred to in Schedule 01.

4.	LIQUIDATED DAMAGES FOR LATE DELIVERY

4.1	For the purpose of paragraph 8.4 of Annex A, BT shall pay the Operator liquidated damages at the rate of £15 per calendar day per 2 Mbit/s. comprised within the Submarine Cable Capacity being accessed by the relevant ISH Link or CSH Link, subject to a maximum of £100,000.  Thereafter, damages shall be at large, subject to paragraph 13 of the main body of this Agreement the limitations therein referred to being inclusive of the maximum amount of the liquidated damages referred to in this provision.

4.2	Where BT has failed to provide and is more than 10 Working Days late, the Operator may cancel the order for Capacity and the provisions of paragraph 8.3 shall not apply.

4.3	The Operator shall neither pay any connection charge nor any rental charge in respect of any ISH Link, CSH Link or Cross Connection during any period when BT is paying or is liable to pay liquidated damages under this provision.