ANNEX A

#### PLANNING AND OPErations

INDEX

A1	Definitions

A2	General

A3	Network Information

A4	Location of Switch Connections

A5	Routing Principles

A6	Interconnect Link Architecture

A7	Capacity Provision

A8	Traffic Forecasts

A9	Capacity Profiles and Advance Capacity Orders

A10	Capacity Orders and Testing

A11	Capacity Order Timescales

A12	Numbering

A13	Technical Review Meetings

A14	Switch Testing

A15	Transmission and Signalling

A16	Performance Standards

A17	Operations

Appendices

A	BT/Operator Network Information

B	Commercial

C	Data Management Amendments

1.	DEFINITIONS

1.1	In this Annex, the Appendices and tables a reference to a paragraph or Appendix, unless stated otherwise is to a paragraph or appendix of this Annex.  Words, abbreviations and expressions have the meanings given in Annex D.

2.	GENERAL

2.1	This Annex details the planning and operational principles for conveying Calls between the BT System and the Operator System and provides the technical basis for the Specifications and for the Manuals. The Specifications define the characteristics of the interface between the BT System and the Operator System.  The Manuals provide procedures for the provision and rearrangement of Capacity (Provisioning Manual), testing (Operational Testing Manuals) and operations and maintenance (Operations and Maintenance Manual).

2.2	This Annex contains provisions on the planning and operation of services provided under this Agreement.  The details of the interconnect between the Parties’ Systems and the plans for future developments relating to interconnect shall be recorded in the Technical Master Plan.  The Customer Service Plan shall contain details of the points of contact within the Operator’s and BT’s organisation.  The Parties shall endeavour to keep these documents up to date.

2.3	If there is a written form for the exchange of information between the Parties pursuant to Annex A, such information shall be exchanged in electronic form.

3.	NETWORK INFORMATION

3.1	Insofar as not previously provided, each Party shall provide to the other the information listed in Appendix A as soon as reasonably practicable and in any event not more than 20 Working Days from the date of this Agreement.

3.2	In addition to the requirements set out in paragraph 4 of the main body of this Agreement, each Party will provide to the other information on any proposed introduction, closure, replacement or modification to any Switch referred to in paragraph 3.1 and where relevant, adjustments to the Number Ranges supported by such Switches as soon as practicable and, in any event, at not less than 4 months prior to the implementation.

4.	LOCATION OF SWITCH CONNECTIONS

4.1	BT Switch Connections

BT shall nominate by written notice to the Operator BT Switch Connections where Calls  are to be handed over from the BT System to the Operator System.

4.2	Operator Switch Connections

BT shall nominate by written notice to the Operator, Operator Switch Connections at an Operator Exchange identified in accordance with paragraph 3.1 at which Calls are to be handed over from the BT System to the Operator System.

4.3	Agreement of Switch Connections

4.3.1	The Parties shall endeavour to agree which Switch Connections shall be used for the conveyance of Calls between the Parties.  In the event that agreement cannot be reached either Party may notify the other in writing of a Dispute

5.	ROUTING PRINCIPLES

5.1	Responsibilities

5.1.1	BT shall route traffic in accordance with the following routing principles:

(a)	routing within BT’s System shall be equivalent to the routing of similar types of traffic for the generality of BT’s Customers including alternative routing where appropriate;

(b)	insofar as practicable routing shall avoid analogue routing within the BT System. The Parties acknowledge that some International Calls and Calls from a Third Party's system may involve analogue routing.

5.1.2	The Parties shall develop and apply network traffic management strategies and procedures to maintain customer service quality and to protect the Parties' Systems as are appropriate. Full details of the network traffic management provisions are set out in the Operations and Maintenance Manual.  These shall include;

(a)	designation of specific Traffic Routes to restore service if a route loss occurs;

(b)	establishment of overflow procedures if there is route congestion;

(c)	establishment of special procedures for busy circuits enabling either diversion of a Call to a different routing or diversion to an agreed tone or message.

5.1.3	The responsibilities for Traffic Types are set out in Table A below.  The responsibilities include provision of Traffic Forecasts, provision of Capacity Profiles and placing of Capacity Orders.

TABLE A

| TRAFFIC TYPE | PARTY HAVING THE RESPONSIBILITIES SET OUT IN PARAGRAPH 5.1.3 |
| --- | --- |
| Calls to Automatic paging services telephone numbers BT to Operator | Operator |
| Calls to bureau based paging services telephone numbers BT to Operator | Operator |
| Calls to voice messaging services BT to Operator | Operator |

5.1.4	The Operator shall ensure that a Route Type is provided on each Interconnect Link for the conveyance of Traffic Types for which that Party has responsibility in accordance with paragraph 5.1.3.  Where a Party wishes to segregate its traffic into different Traffic Types it may order additional Route Types in accordance with Schedule 1 or 2 as appropriate.

5.2	Interconnect Traffic Route Structure

5.2.1	The Parties shall establish Interconnect Links to convey Calls between the BT System and the Operator System on Traffic Routes between each Operator Switch Connection and at least one BT Switch Connection.

5.2.2	Traffic Routes shall be set up to convey traffic from the BT System to the Operator System only.

5.2.3	The Operator shall assign Traffic Streams (identified by destination) to a particular Traffic Route. The Operator shall, subject to paragraph 5.4 below, advise BT which Traffic Streams are assigned to which Traffic Route. Unless otherwise agreed, each Party shall use its reasonable endeavours to ensure that traffic is directed in accordance with the advice given to the other. Traffic so routed is primary traffic as described in paragraph 5.4.

5.2.4	Where there is a Traffic Route to or from a Switch Connection and notification has been given by either Party that the relevant Switch Connection is to be removed then no additional Capacity shall be provided on that Traffic Route save that agreed in the Advance Capacity Order current on the date when notification was given.

5.3	Interconnect Traffic Route Dimensioning

5.3.1	Traffic Routes shall be Fully Provided to carry only the traffic for which Capacity has been ordered in accordance with this Annex and such Traffic Routes shall be operational at no worse than the required grade of service.

5.4	Interconnect Routing Rules

5.4.1	BT to Operator traffic

BT may convey Traffic Streams in any combination of Fully Provided Traffic Routes from BT Switch Connections to Operator Switch Connections only in accordance with routing plans agreed by the Parties for those Traffic Streams.

All traffic delivered in accordance with this paragraph shall be considered to be primary traffic

5.4.2	Overflow Traffic

Traffic overflowing as part of an agreed routing plan will be deemed to be included in the provisions of paragraphs 5.4.1. and shall be considered to be  primary traffic .

5.4.3	Other Traffic

Traffic that is delivered other than as detailed in paragraphs 5.4.1 and 5.4.2 may be given a lower priority than primary traffic and should only be delivered after agreement by the parties in accordance with the provisions of the Operations and Maintenance Manual.

5.5	Interconnect Route Diversity and Security

Where either Party requests physical route diversity it shall take account of the Signalling Links and the Traffic Routes.

5.6	Record of Arrangements

The Parties shall record the physical arrangements and Number Ranges, or part thereof, for each of the Traffic Routes on an Interconnect Link. Information shall be provided in accordance with the Provisioning Manual and a record of these arrangements shall be maintained in the Technical Master Plan.

6.	INTERCONNECT LINK ARCHITECTURE

Paging Interconnect Links shall be provided by BT, subject to the provisions of Schedules 1 or 2 and the Specifications.

7.	CAPACITY PROVISION

7.1	Initial Interconnection

Prior to the first Capacity Order under this Agreement:-

7.1.1	the Operator shall provide BT with the following information: -

(1)	details about the proposed Operator Exchange or Exchanges software build level;

(2)	details of the services (as in the Schedules) which the Operator requires on the first Ready for Service Date;

(3)	Number Ranges served by and that may be accessed via the Operator System;

(4)	a statement of the conformity of the Operator System to the Specifications;

(5)	the first Capacity Profile (as described in paragraph 9 below);

(6)	the first Traffic Forecasts (as described in paragraph 8 below);

(7)	proposed Traffic Routes, by Number Ranges (in the form set out in the Provisioning Manual);

7.1.2	BT shall provide the Operator with the following information:-

(1)	proposed signalling and testing requirements within the time scale set out in paragraph 14.2 below;

(2)	details of the relevant Operator Switch Connections and BT Switch Connections nominated by BT (if any), within 20 Working Days of a written request by the Operator;

(3)	proposed Traffic Routes by Number Ranges if the Operator is planning to use more than one Operator Switch Connection at the first Ready for Test Date within 20 Working Days of a written request by the Operator.

7.1.3	The information specified in paragraphs 7.1.1 and 7.1.2 above shall be exchanged in the manner detailed from time to time in the Provisioning Manual. The Parties shall hold such meetings as are required for this exchange of information.

7.2	Where the Operator is seeking an initial interconnection to the BT System to transfer traffic generated by an established customer, the Operator shall provide BT not less than 20 Working Days prior to the first planning meeting referred to in paragraph 7.1.3 above with the following information:-

(1)	traffic volumes from the BT System to the Operator System for each proposed BT Switch Connection; and

(2)	specific requirements for the transfer of the Operator's existing Number Ranges.

8.	TRAFFIC FORECASTS

8.1	General

Traffic Forecasts shall be used by the Parties for planning sufficient switch and distributive network capacity to meet subsequent Capacity Order requirements. While the Parties shall use reasonable endeavours to provide accurate Traffic Forecasts, they shall not be legally binding except to the extent stated otherwise in this Agreement.

8.2	Traffic Forecast Content

8.2.1	Traffic Forecasts for each BT Switch Connection shall be prepared by the Operator being the aggregate of the Traffic Types listed in Table A of this Annex for which the Operator is stated to have the responsibility as defined in paragraph 5.1.3.

All Traffic Forecasts shall be in the form set out in the Provisioning Manual.

8.3	Forecasting Periods

8.3.1	Traffic Forecasts shall be on a rolling basis for a period of three years.

8.3.2	A Traffic Forecast provided by the Operator to BT at yearly intervals for each Switch Connection pursuant to paragraph 8.2 shall be made available at least 15 Working Days before the Technical Review Meeting immediately before the Traffic Forecast period next following. These Traffic Forecasts shall (subject as provided below) cover the following three years broken down for each four month period for the first year.

8.3.3	If the operator's Traffic Forecast for a category of traffic for any twelve month period at any Switch Connection has changed 10 per cent. or more since the last Traffic Forecast or where any additional Switch Connection is proposed by the operator during the next twelve months, then the changed Traffic Forecast shall be notified not later than the next Technical Review Meeting following such change or adoption of such proposal.

8.3.4	During the first two years after the first Ready for Service Date, the Operator may supply Traffic Forecasts covering only two years. Thereafter, such Traffic Forecasts shall cover a period of three years.

8.4	Traffic Forecast Information

8.4.1	Traffic Forecasts supplied pursuant to paragraph 8.3 shall:

(a)	be in terms of day and evening busy hour Erlangs, busy hour call attempts and average call holding time. The day busy hour shall be within the period 0800 to 1800 hours and the evening busy hour shall be within the period 1800 to 0800 hours except where agreed otherwise;

(b)	identify the time of the busy hour for a Traffic Route to be configured pursuant to paragraph 8.1;

(c)	be in the form set out in the Provisioning Manual,

(d)	be authorised and signed at an appropriate level by the originating Party and acknowledged by the receiving Party.

8.5	Forecasting Evolution

8.5.1	The Parties may agree to modify the procedures in this paragraph 8 if and when additional Traffic Routes and/or Traffic Streams are identified or if presently planned Traffic Routes are subdivided.

8.5.2	The forecasting procedures shall be reviewed at the Technical Review Meeting to discuss routing and forecasting matters and to agree changes and appropriate timings for their implementation.

9.	CAPACITY PROFILES AND ADVANCE CAPACITY ORDERS

9.1	Before placing Capacity Orders, the Operator shall supply BT with Capacity Profiles in relation to Capacity Provision and Capacity Re-arrangement at all existing and proposed Switch Connections.  In paragraphs 9 and 10 and subject to paragraph 11.4, Capacity Rearrangement may only be initiated by the Purchaser of the Capacity which is being rearranged.  Each Capacity Profile shall encompass all the Operator's requirements for Capacity and shall be identified for each relevant Switch Connection.

9.2	Capacity Profiles for Capacity Provision and Capacity Re-arrangement shall be provided on forms as described in the Provisioning Manual.  The first four months of each Capacity Profile agreed in accordance with paragraph 9.4 shall form an Advance Capacity Order pursuant to paragraph 10.

9.3	Each Capacity Profile shall be presented to the other Party at four-month intervals, not less than 15 Working Days before the relevant Review Meeting.

9.4	Each Capacity Profile shall be reviewed at the relevant Technical Review Meeting. Where agreed it will be signed within 5 Working Days of such agreement by the authorised representatives of each Party to signify their intention to commit to the Advance Capacity Order.

9.5	If the Parties fail to agree a Capacity Profile (or part thereof), then either Party may notify the other in writing of a Dispute.

9.6	Those portions of an Advance Capacity Order that have not been agreed shall be a Dispute. Paragraph 10 shall apply to those parts that have been agreed (and signed by the Parties).

10.	CAPACITY ORDERS AND TESTING

10.1	Capacity Ordering

10.1.1	In an ACO Period the Operator may, during the period ending two years after the first Ready for Test Date, in respect of any Operator Switch Connection nominated by BT (for traffic handed over from the BT System to the Operator System), place Capacity Orders up to the amount of the aggregate Capacity Provision (i.e. Capacity to be provided plus the provide element of Capacity to be rearranged) shown in the relevant Advance Capacity Orders for each such Switch Connection plus the greater of (a) 10 per cent. of such Capacity and (b) 4 x 2Mbit/s units of Capacity where such Capacity is shown as being greater than zero. Within these limits BT shall provide such Capacity that it is to provide within the time scales laid down in paragraph 11.

10.1.2	In an ACO Period the Operator shall place Capacity Orders for a minimum of 80 per cent. of the aggregate Capacity specified in the relevant Advance Capacity Orders for each Switch Connection.  Failure by the Operator to place such Capacity Orders shall result in pre-payment charges being paid by the Operator in accordance with paragraph 1 of Appendix B. Pre-payment charges arising with respect to a specific Switch Connection shall be accepted as pre-payment for future Capacity Orders in respect of Capacity for the same Switch Connection placed within 12 months of the end of the relevant ACO Period (thereby being credited against any charges payable by the Operator in respect of such Capacity Order(s)).

10.1.3	If no Advance Capacity Order is provided or Capacity Orders are placed by the ordering Party in excess of the relevant limit in paragraph 10.1.1:-

(1)	the Supplier shall not be obliged to meet the time scales set out in paragraph 11 for that element of the Capacity Order in excess of the relevant limit;

(2)	the time scales shall be agreed in writing for the provision of excess Capacity and the Ready for Test Dates shall be not later than 6 months after the date of receipt of a relevant Capacity Order; and

(3)	the Supplier of Capacity in excess of the relevant limit may raise charges to cover any additional costs that it may incur arising directly from the provision of Capacity outside the normal planning process. Neither Party will incur such additional costs or provide Capacity outside the Advance Capacity Order process without the prior written approval of the other.

10.1.4	Each Capacity Order will be in the form set out in the Provisioning Manual  and shall include all the information required by that form.

10.1.5	The tributary allocation charts as set out in the Provisioning Manual, shall be amended by BT, unless agreed otherwise and included in the Technical Master Plan.

10.2	Testing Timetables

10.2.1

(1)	BT and the Operator shall commence testing Capacity not later than 25 Working Days after the relevant Ready for Test Date. Unless agreed otherwise, the Purchaser shall provide the Supplier with a minimum of 15 Working Days notice of the date on which testing should commence.  The Parties shall co-operate to ensure that testing is reasonably spread across the 25 Working Days following the date on which the Capacity is made ready for testing.  The Parties shall use reasonable endeavours to complete testing in the shortest appropriate time.

(2)	If for any reason the Purchaser expects to be unable to proceed jointly with the Supplier in testing of any Capacity, the Purchaser shall notify the Supplier in writing as soon as possible and where such notice is less than 5 Working Days before the scheduled Ready for Test Date, the provisions set out in paragraph 3 of Appendix B of this Annex shall apply. Testing shall then be re-scheduled by agreement and commenced on such re-scheduled date.  In the event of testing failing the Parties shall use reasonable endeavours to ensure testing is recommenced and concluded as quickly as possible.

(3)	The Parties shall agree the required test facilities.  The Parties shall make such test facilities available from the date required by the other Party.  If specified test facilities are missing or incorrect, Capacity Orders or Data Management Amendments will not commence until the facilities are made available.

(4)	If the Party providing Capacity fails to commence testing Capacity by a date 25 Working Days after the relevant periods of time specified in paragraph 11 below, then (without prejudice to the other Party's rights and remedies under the Agreement) the Party providing the Capacity Party shall pay, on demand, to the other Party liquidated damages for such delay calculated in accordance with the provisions of paragraph 2 of Appendix B for up to a maximum of 25 Working Days.

(5)	If the Parties fail to agree an acceptable date for the testing of the Capacity (pursuant to 10.2.1 (2)) either Party may notify  the order in writing of a Dispute.

10.2.2	On the Ready for Test Date the Supplier shall notify the Purchaser that such Capacity is Ready for Testing. Such notice shall be in the form  set out in the Provisioning Manual.

10.2.3	As soon after the Ready for Service Date as is practical, both Parties shall certify that the Capacity has been satisfactorily tested by signing the certificate set out in the Provisioning  Manual.

10.3	Testing and Commissioning Procedures

The Parties shall jointly test the Capacity and any service specific items and make any necessary adjustment to ensure that the Capacity meets the agreed performance standards. Details of the testing procedures are included in the Operational Testing Manuals.

11.	CAPACITY ORDER TIMESCALES

11.1	Timescales for Provision of Capacity

The timescales for the provision of Capacity Ready for Testing are subject to the provisions set out in Schedules 1 and 2 (as appropriate) and paragraph 10.1.3. The time scales are set out in paragraphs 11.1.1 to 11.1.5 inclusive and are concurrent with those set out in paragraph 11.1.1:-

11.1.1	Capacity Provision at a Switch Connection for a Traffic Route passing through a new Point of Connection on a Paging Interconnect Link:-

6 months following the date of receipt of the relevant Capacity Order.

11.1.2	New Traffic Routes between Switch Connections through an established Point of Connection using the same technology, including the provision of new Traffic Routes by way of additional interconnection from a particular Operator’s site:-

65 Working Days following the date of receipt of the relevant Capacity Order.

11.1.3	Route Augmentation using the same technology, including the provision of additional interconnection between the same Switch Connections:

25 Working Days following the date of receipt of the relevant Capacity Order.

11.1.4	Capacity Re-arrangements involving the provision of new Traffic Routes already in service at the relevant Switch Connection:-

65 Working Days following the date of receipt of the relevant Capacity Order.

11.1.5	Capacity Re-arrangements between established Switch Connection involving Route Augmentation already in service at the relevant Switch Connection:-

25 Working Days following the date of receipt of the relevant Capacity Order.

11.2	Removal of Capacity

11.2.1	Subject to the relevant Schedules if either Party requires the removal of Capacity it paid for (or for which payment is due by that Party under this Agreement), an order identifying the Capacity and the date from which it is no longer required (a "removal order") may be placed by that Party on the other Party. The Party receiving the removal order will remove the Capacity not later than 30 Working Days from the date of receipt of the removal order. However this time scale may be extended if agreed in writing by the Parties in order to coincide with a planned programme of work.

11.2.2	If either Party requires the removal of Capacity paid for by the other Party (or for which payment is due by the other Party under this Agreement) then a written request shall be sent to the other Party detailing the requirement and giving the reasons for its removal. If the Party receiving the request agrees to the removal of the Capacity such Party shall issue a removal order to the Party requesting the removal. This Capacity will then be removed by the Supplier within 30 Working Days from the date of receipt of the removal order or such other date as the Parties may agree. Agreement to a removal request shall not be withheld where it can be demonstrated that the requirement for the Capacity is not supported by the current agreed Traffic Forecast. However, if the Party receiving the request still does not agree then that Party may notify the other in writing of a Dispute and Capacity shall not be removed until agreement has been reached or the Dispute is resolved.

11.2.3	A removal certificate shall be issued to the Party who paid (or who is responsible for payment) for the Capacity on completion of the removal work.

11.2.4	For the avoidance of doubt, if payment for Capacity removed pursuant to this paragraph 11 has not been made at the time of such removal such payment shall remain due and payable.

11.2.5	If pursuant to this paragraph 11.2 Capacity is removed, a proportion of the relevant connection charge shall be repaid to the Party requesting the removal. The amount to be repaid shall be that set out in paragraph 4 of Appendix B to this Annex.

11.3	Order Amendment

11.3.1	Capacity Orders may be amended as follows:-

(a)	Capacity Orders which incorporate agreed amendments requested by the Supplier or are corrections of simple typing mistakes which do not affect the Ready for Test Date nor the implementation by the Supplier will not incur any charges.

(b)	Capacity Orders may be withdrawn free of charge up to 10 Working Days from receipt of the relevant Capacity Order by the Supplier. Capacity Orders, including any placed in the last 10 Working Days of an ACO Period which bring the total orders up to 80% of the Capacity specified in the ACO, and  subsequently cancelled within 10 Working Days of placing those orders shall not be included in the amount of Capacity ordered in the relevant ACO Period.

(c)	Capacity Orders may be amended at a charge, details of which  are specified from time to time in the Carrier Price List, up to 10 Working Days from receipt of the Capacity Order by the Supplier. The original date that the Capacity Order was placed will apply for the purpose of identifying the Ready for Test Date.

(d)	Capacity Order amendments received after 10 Working Days from receipt of the original Capacity Order by the Supplier which do not have a material affect on the order (i.e. editorial change, data build change only, without change of Supplier's Exchange or other change which the Parties reasonably agree does not materially affect the Capacity Provision within the stated time scales) will be accepted by the Supplier without a change in the Ready for Test Date at a charge which is set out in the Carrier Price List.

Where an amendment involves data build changes, the order amendment must be received by the Supplier not later than 10 Working Days before the Ready for Test Date

11.3.2	If a Capacity Order is amended other than as set out in paragraph 11.3.1 (a) (b) (c) or (d) above, and the amendment is to reduce the Capacity originally ordered, the Purchaser shall pay the charge set out in paragraph 5 of Appendix B. The Supplier shall accept such an amendment without a change in the Ready for Test Date.

11.3.3	Apart from as detailed in paragraph 11.3.1 (a) or (b) and paragraph 11.3.2, no amendments of Capacity Orders will be accepted after 10 Working Days from receipt of the Capacity Order by the Supplier.

11.3.4	The Supplier may extend a Ready for Test Date where there have been two or more order amendments to any matter included in the order form set out in the Provisioning Manual during the first 10 Working Days following receipt of a Capacity Order.

11.4	Capacity Rearrangement

Capacity Rearrangement may be requested as follows:-

(a)	The Purchaser may request in writing Capacity Rearrangement, and the Supplier shall carry out the necessary work in accordance with the time scales set out in paragraph 11.1.

(b)	If the Supplier requests Capacity Rearrangement of Capacity previously provided by the Supplier, the Supplier shall notify the Purchaser in writing giving reasons for its request. If the Purchaser accepts the request it shall confirm the same in writing. The Supplier shall then at its own expense carry out the Capacity Rearrangement in accordance with the request. The Purchaser shall not unreasonably withhold or delay consent to such request provided that arrangements are made, in accordance with this Agreement where applicable, to ensure that no loss is caused to or suffered by the Purchaser as a result of the Capacity Rearrangement.

(c)	The Supplier shall issue a certificate to the Purchaser on completion of work carried out pursuant to paragraph 11.4 (a) or (b). The Purchaser may submit requests for Capacity Rearrangement in writing to the Supplier at any time after the earlier of Ready For Service Date or one month after Ready For Test Date.

(d)	On completion of a Capacity Rearrangement the original Capacity is deemed terminated and replaced by the Rearranged Capacity.

11.5	Cancellation of Order

At any time prior to the Ready for Test Date a Party may by written notice to the other cancel a Capacity Order it placed. If a Capacity Order is cancelled more than 10 Working Days after placement of the Capacity Order, the cancelling Party shall pay a cancellation charge as set out in paragraph 6 of Appendix B. Capacity Orders, including any placed in the last 10 Working Days of an ACO Period which bring the total Capacity Orders up to 80% of the Capacity specified in the ACO, and subsequently cancelled within 10 Working Days of placing those orders shall not be included in the amount of Capacity ordered in the relevant ACO period.

11.6	General

The Party in receipt of a Capacity Order shall make the relevant Capacity available and Ready for Testing within the time scale specified in paragraph 11.1.  The receiving Party shall notify the ordering Party of such receipt within one Working Day of receipt.

12.	NUMBERING

12.1	Numbering Orders

Subject to paragraph 8 of the main body of the Agreement, the Party in receipt of a request by the other Party to implement numbering orders for Number Ranges and associated changes to data in the System shall do so in accordance with Appendix C.

12.2	Number flow

The Parties shall use and BT shall convey to the Operator telephone numbers in the national number format from time to time in force.

12.3	Number Information

At appropriate intervals each Party shall inform the other of all numbering additions and changes to Number Ranges within the BT System or Operator System that may affect Call routing.

13.	TECHNICAL REVIEW MEETINGS

13.1	The first Technical Review Meeting shall occur at either Party's written request not later than one month from the initial in-service date. Subsequent Technical Review Meetings shall occur at no greater than four monthly intervals thereafter, not less than 5 Working Days prior to the start of the relevant four month period next following.

13.2	At Technical Review Meetings the Traffic Forecast referred to in paragraph 8 shall be discussed and the Capacity Profiles referred to in paragraph 9 shall be agreed.

14.	SWITCH TESTING

14.1	Testing Principles

Each Party shall have the right to require testing of the other Party’s Switch(es) to confirm correct interworking. The testing varies according to the Switch manufacturer, the software build level and the method of implementation. The scope of the testing shall follow the relevant ITU-T guidelines.

14.2	Testing Information

14.2.1	Not later than 10 Working Days after a written request by one Party, the other Party shall provide an initial indication of the level of testing required, having been given details of Switch hardware and software build level, services and a statement of conformity of the Party's System to the specifications as detailed in paragraph 7.1.1. The Parties shall agree exact test requirements and timescales.

14.2.2	If, owing to the default of one of the Parties the testing procedures vary substantially from the test procedures agreed pursuant to this paragraph, the other Party may levy a reasonable charge for its direct costs resulting from such default.

14.3	Where BT requests testing, the following provisions shall apply:

14.3.1	Reduced Level of Testing

A Switch proposed as an Operator Exchange, of the same type previously successfully tested for the same type of interconnect, at the same software build will need the minimum of testing. Commissioning and Acceptance Testing ("CAT") of such an Operator Exchange may take approximately four weeks. For additional such Operator Exchanges, Traffic Routes and Capacity interconnected to the BT System, operational testing at a suitable level will take place as described in the Operational Testing Manual.

14.3.2	Intermediate Level of Testing

A Switch proposed as an Operator Exchange of a type previously successfully tested at a different software build or implementation, will need additional testing, if such different build or different implementation could reasonably be considered likely to adversely affect Call processing. Both Network Inter- working Tests ("NIT") using BT captive models and CAT will be required, which may take approximately eight weeks.

14.3.3	Full Switch Testing

A Switch proposed as an Operator Exchange or major build change of an existing Operator Exchange not previously successfully tested for the same type of interconnect shall require Interface Validation Testing ("IVT") by the Operator, using a protocol simulator, which may take the Operator about six weeks.  The total testing duration for IVT, NIT and CAT may be approximately 14 weeks. This period of time (other than for the testing specified in paragraph 14.3.1) may, subject to the availability of the necessary BT captive models at the Operator's written request, be concurrent with the time scale set out in paragraph 11.1.1 above.

14.4	Where the Operator requests testing it will employ testing no more onerous and be of no greater duration than those set out in paragraph 14.3. .

14.5	In the event that the Parties fail to agree any test procedure and/or time scale, then either Party may notify the other in writing of a Dispute.

15.	TRANSMISSION AND SIGNALLING

15.1	Transmission

Interconnection of the BT System and the Operator System shall be based on technology described in Schedules 1 or 2 operating at 2Mbit/s in accordance with the Technical Interface Specification.

15.2	Synchronisation

15.2.1	The Parties shall comply with the applicable synchronisation provisions of the Generic Electrical and Physical Interface Specification. Where requested, BT shall supply synchronisation from the highest order available within the relevant part of the BT System.

15.3	Signalling

The Parties shall comply with the applicable signalling provisions of the Technical Interface Specification.

15.4	Circuit Selection

The following circuit selection protocols shall apply:-

(1)	Circuit Selection protocols

BT shall use the forward sequential protocol, starting from the first traffic circuit and selecting sequentially until a free circuit is found.

(2)	Direction of search

From BT to Operator, direction of traffic circuit 1 to Z.

15.5	Numbering of traffic circuits

Circuits shall be identified pursuant to the Technical Interface Specification.

15.6	Answer Signals

The Operator shall provide BT with details of the timing and circumstances for the provision of all Answer Signals sent from the Operator System including those for all types of Calls.

16.	PERFORMANCE STANDARDS

16.1	General

16.1.1	The Parties shall co-operate to maintain the overall quality of the conveyance of Calls and adopt the general principles on standards, techniques and methodology for the achievement of quality in telecommunications networks and services contained in the relevant ITU-T standards.

16.1.2	The Parties shall agree operational management strategies to protect service quality and to alleviate short term overloads due to abnormal conditions and address congestion due to under provisioning.

16.2	Call Service Performance

16.2.1	The Parties shall agree relevant service parameters and measurements for the monitoring of service performance. These will typically include sample sizes, frequency, time and performance threshold criteria. This data includes actual measurements of exchange or route busy hour traffic (Erlangs), lost communications (overflows), number of Calls alternatively routed, Answer Seize Ratio ("ASR") and busy hour call attempts.  This shall be exchanged on a monthly basis. Exception reports shall be provided where standards have not been met or if performance thresholds have not been attained. The Parties shall use their reasonable endeavours to detect and correct adverse trends before they result in adverse performance.

16.2.2	If a service performance problem is identified, the Parties shall exchange information about the problem, including the information set out in paragraph 16.2.1 above together with the following information:-

(1)	critical levels for traffic and unsuccessful Call attempts;

(2)	information on sources of traffic;

(3)	information on timing synchronisation and slippage rates.

16.2.3	The Parties shall use reasonable endeavours to identify and resolve the problem and if the Parties fail to resolve the problem it shall become a Dispute.

16.3	Interconnect Link Service Performance

Transmission performance standards over a Paging Interconnect Link shall be based on availability, errored seconds and severely errored seconds over a one month period.

16.4	Information on Performance Standards

Each Party ("the first Party") shall provide to the other, as the other Party may reasonably require, information on the performance standards of the first Party's System from time to time for the purpose of the other Party verifying the performance by the first Party of its obligations under this Agreement, including without limitation the performance standards referred to in this paragraph 16, routine maintenance time scales and fault repair times as set out in the Operations and Maintenance Manual.

17.	OPERATIONS

17.1	General

17.1.1	Each Party shall be responsible for the safety and operation of its own System.

17.1.2	The Operations and Maintenance Manual details the responsibilities, methods and procedures for the operation and maintenance of the interconnection of the Systems.  Details of contacts are given in the Customer Service Plan.

17.2	Fault Identification and Reporting

17.2.1	Each Party will advise its Customers to report all faults to its fault reporting centre.

17.2.2	If a Party's Customer reports a fault to the fault reporting centre of the other Party, that Party will inform the Customer of the correct number to which to report the fault.

17.2.3	If a major fault occurs which affects both Parties' Systems, initial responsibility for identifying the fault rests with the Party who first becomes aware of the fault.

17.2.4	If a Party identifies a fault occurring in its System which may have an adverse effect on the other Party's System, the first Party shall promptly inform the other Party of the actions being taken to resolve the problem.

17.2.5	Each Party shall bear the costs of its own fault reporting centre.

17.3	Speed of Response

The Parties shall develop and record in the Operations and Maintenance Manual, a series of agreed response times for different fault conditions, response time being the time from the reporting of the fault until remedial action begins.

17.4	Service Restoration

The service restoration procedures set out in the Operations and Maintenance Manual are based on the following:-

(1)	service restoration shall take priority over the clearance of faults not affecting service unless it is agreed that a fault be treated as a lower priority;

(2)	the relevant Party shall automatically bring in available standby Capacity and/or carry out network management actions to restore service;

(3)	the relevant Party shall observe equipment alarms and carry out testing to identify the nature and location of the fault in co-operation with the other Party as necessary;

(4)	if possible, the relevant Party shall rectify the fault immediately, if not, the other Party must be notified and kept informed of progress on a regular basis;

(5)	if temporary repairs are made, the other Party must be informed and given the estimated timescale and service impact of full repair; and

(6)	priority shall be given by the relevant Party to faults with the highest service loss impact on the number of Customers or the volume of traffic affected.

17.5	Restoration Times and Procedure

Indicative restoration times and procedures are set out in the Operations and Maintenance Manual.

17.6	Planned Maintenance/Works

17.6.1	Each Party shall provide not less than 3 Working Days notice of any planned maintenance which may affect the other's System. Each Party shall use its reasonable endeavours to minimise disruption and where possible provide alternative routing at no charge to the other Party for a reasonable period of time with respect to the cause of the disruption.

17.6.2	If a Party reasonably considers that the interconnection arrangements are at risk and maintenance repair activity is essential, that Party shall give as much advance notice to the other Party as is reasonably practicable.

APPENDIX A

BT/OPERATOR NETWORK INFORMATION

1.	OPERATOR INFORMATION

Names and Addresses of all Operator Exchanges within the area covered by the BT System.

2.	OPERATOR NUMBERING INFORMATION

2.1	Number Ranges of Operator Exchanges within the area covered by the BT System

3.	BT INFORMATION

3.1	Names and addresses of BT Tandem Exchanges

3.2	Names and addresses of BT DLTEs

3.3	Names and addresses of BT DLEs

4.	BT NUMBERING INFORMATION

4.1	Number Ranges of BT DLEs, BT DLTEs and of those exchanges which are connected by a DLE alone to the remainder of the BT System.

4.2	For each BT DLE and BT DLTE, the names of the BT Tandem Exchanges which are the Parent BT Exchanges.

4.3	For each BT DLE, the name of the BT DLTE.

APPENDIX B

COMMERCIAL

1.	PRE-PAYMENT CHARGES

1.1	If either Party (the “Defaulting Party”) in any ACO Period, places Capacity Orders on the other Party for less than the minimum Capacity specified pursuant to paragraph 10.1.2 of Annex A, then the Defaulting Party shall pay to the other Party not later than 30 Working Days of the end of such ACO Period the prepayment charges calculated in accordance with paragraph 1.2 of this Appendix.

1.2	Prepayment charges are calculated in accordance with the following formula:-

A	=	((80% x B)-C) x D

Where:-

A	is the prepayment charge payable:

B 	is the aggregate Capacity Provision (by number of units 2Mbit/s Capacity) specified for the relevant ACO Period in the relevant Advance Capacity Order:

C	the Capacity (by number of units of 2Mbit/s Capacity) ordered during such ACO Period not including cancellations of Capacity Orders made during or after the relevant ACO Period:

D	is half the Intrabuilding Link connection charge appliable for the ACO Period, as detailed in the Carrier Price List from time to time.

2.	LIQUIDATED DAMAGES IN RESPECT OF TESTING

CAPACITY

2.1	The liquidated damages referred to in paragraph 10.2.1(4) of Annex A shall be 1.5% of Intrabuilding Link connection charge specified from time to time in the Carrier Price List for each 2Mbit/s unit of Capacity which is subject of the relevant Capacity Order, multiplied by the number of Working Days calculated from a date 25 Working Days after the date on which the Capacity should have been Ready for Testing pursuant to paragraph 11 of Annex A, except that no liquidated damages shall be payable if the Purchaser causes a delay to the Ready for Service date by failing to complete the testing of the Capacity in the time period set out in paragraph 10.2.1(1) of Annex A.

3.	CANCELLATION CHARGES IN RESPECT OF LATE NOTIFICATION OF TESTING

3.1	If the notice by the Purchaser, pursuant to paragraph 10.2.1(2) of Annex A is given less than five Working Days before the scheduled Ready for Test Date, the Purchaser shall pay, on demand, a cancellation charge of 20% of the relevant connection charge, or charges, specified from time to time in the Carrier Price List and referred to in Schedule 1 or 2, as  appropriate.

4.	REMOVAL OF CAPACITY

4.1	If, pursuant to paragraph 11.2 of Annex A, Capacity is removed a proportion of the relevant connection charge shall be repaid to the Party requesting the removal.  The amount to be repaid shall be

R = (0.466 x C) - (0.466 x C x Y)

10

where:

R	is the amount to be repaid

C	is the relevant connection charge referred to in Schedule 1 or 	2, as appropriate, and specified from time to time in the Carrier 	Price List

Y	is the number of years between the Ready for Test Date for 		the relevant Capacity and the date of the removal order rounded 	up to the next whole year.

5.	ORDER AMENDMENT

5.1	The charge payable pursuant to paragraph 11.3.2 of Annex A shall be 50% of the difference between the connection charges specified from time to time in the Carrier Price List, for the originally ordered and the amended Capacity Order.

6.	ORDER CANCELLATION

  - The charge payable pursuant to paragraph 11.5 of Annex A shall be 50% of the applicable connection charge specified from time to time in the Carrier Price List.

APPENDIX C

##### DATA MANAGEMENT AMENDMENTS

1.              Introduction

1.1	This Appendix applies to Data Management Amendments made pursuant to a request by a Party (the “Requesting Party”) in the System of the other Party (the “Changing Party”).

1.2	The Parties acknowledge that no individual charges shall be invoiced by either Party in respect of a Data Management Amendment requested under paragraphs 2 to 5 of this Appendix (inclusive).

2	New Numbering Requirements

2.1	The initial provision by the Changing Party to the Requesting Party of a service or facility (which the Changing Party provides under interconnect agreement(s) to Third Party Operator(s)) requiring data changes on a Switch necessary for a group of Call destinations for which the same retail charges apply (“Charge Band”) to set up Traffic Streams for Number Ranges and Access Codes, which data changes apply to:

(a) a new AFN;

(b) a new NNG

(c) discrimination of a single D digit, or DE digit on a new NNG;

(d) discrimination of more than one D digit, or DE digit on a new NNG;

(e) a new Access Code;

(f) a new Targeted Transit Prefix Code;

(g) initial service establishment for Number Portability Prefix Codes (both geographic and non-geographic).

2.2	Additional provision by the Changing Party to the Requesting Party of a service or facility requiring data changes on a Switch necessary for a group of Call destinations for which the same retail charges apply (“Charge Band”) to set up Traffic Streams for Number Ranges and Access Codes, which changes do not involve routing changes and which data changes apply to:

(a) an additional AFN;

(b) an additional NNG within a geographic number range or non-geographic mobile number range;

(c) an additional NNG other than in paragraph 2.2 (b) above;

(d) additional discrimination of a single D digit, or DE digit on a new NNG;

(e) additional discrimination of more than one D digit, or DE digit on a new NNG;

(f) additional discrimination of a single D digit, or DE digit on an existing NNG;

(g) additional discrimination of more than one D digit, or DE digit on an existing NNG;

(h) an additional Access Code;

(i) a subsequent Targeted Transit Prefix Code.

3	Traffic Stream Changes on a Switch

3.1	The data changes on a Switch necessary to alter the destination of an existing Traffic Stream, which do not require:

(i)	Interconnect Link changes, or

(ii)	the provision of an additional Interconnect Link, or

(iii) any additional numbering (whether Number Range, access code or prefix code),

which changes apply to an existing:

(a) AFN;

(b) NNG;

(c) NNG with routing discrimination down to one or more D digit combinations;

(d) NNG with routing discrimination down to one or more DE digit combinations;

(e) Access Code;

(f) Targeted Transit Prefix Code.

3.2	The data changes on a Switch necessary to alter the destination of an existing Traffic Stream, which data changes require Interconnect Link changes, the provision of an additional Interconnect Link and/or additional Switch capacity, and any additional numbering changes to an NNG or AFN, which data changes apply to an existing:

(a) AFN;

(b) NNG;

(c) NNG with routing discrimination down to one or more D digit combinations;

(d) NNG with routing discrimination down to one or more DE digit combinations;

(e) Access Code;

(f) Targeted Transit Prefix Code.

4	Changes to Charge Band Allocation

4.1	The data changes on a Switch necessary because of a change required by the Requesting Party to the charges payable by the Parties under a relevant Schedule, to relocate the Number Ranges from an allocated Charge Band to another current Charge Band which data changes apply to:

(a) NNG(s);

(b) NNG(s) with charging discrimination down to one or more D digit combinations;

(c) NNG(s) with charging discrimination down to one or more DE digit combinations.

5	Differential D Digit Charging Discrimination

5.1	The data changes on a Switch necessary to expand an existing NNG to an NNG with charging discrimination to a minimum of two D digits.

6.	Orders for Data Management Amendments

6.1	The Requesting Party shall submit to the Changing Party a written request for a Data Management Amendment using the appropriate form and specified pre-requisites contained in the New Services Manual (or its successor document) to provide the Changing Party with the information necessary to enable it to produce an implementation programme.

7.	Implementation Process

7.1	Subject to paragraphs 7.3, 7.4 and 7.6, if the Requesting Party requires an implementation programme for Data Management Amendments being a date of completion not less than:

7.1.1	30 Working Days from the date of acceptance of the order for Data Management Amendments set out in paragraphs 2.2 (a), (b), (f), (g) and 3.1of this Appendix ;

7.1.2	40 Working Days from the date of acceptance of the order for Data Management Amendments set out in paragraphs 2.1, 2.2 (c), (d), (e), (h), (i), 3.2, 4 and 5 of this Appendix ;

the Changing Party shall achieve such completion date.

7.2	The Parties may, on an occasional basis, agree in writing a completion date for provision of a service or facility which date is less than 30 Working Days or 40 Working Days under paragraph 7.1.1 or paragraph 7.1.2, respectively. The Changing Party shall use its reasonable endeavours to achieve such date of completion.

7.3	Exceptionally, if the Changing Party is unable to meet a requested date it shall notify the Requesting Party as soon as practicable, and the Parties will enter into good faith negotiations to agree an alternative date.

7.4	If the Requesting Party requires data changes requiring a series of Data Management Amendments which necessitate  timing and/or sequencing of data changes for the implementation of such Data Management Amendments, including, for the avoidance of doubt, the provision or re-arrangement of any network element or capacity in either Party’s System, the Requesting Party shall provide to the Changing Party necessary information to enable the Changing Party to produce an implementation programme.

7.5	Pursuant to paragraph 7.4 above, the Changing Party shall use reasonable endeavours to minimise the time taken to complete the implementation programme, taking into account the timescales set out in paragraph 7.1 above for individual Data Management Amendments together with the dependencies (in terms of timing and/or sequencing of data changes) between each Data Management Amendment within the implementation programme.

7.6	As soon as practical and not later than two Working Days from the date of receipt, the Changing Party shall acknowledge in writing receipt of the order. The Changing Party shall proceed with the implementation programme of the Data Management Amendment and complete such programme not later than the agreed date of completion provided that the following pre-requisites are met:

7.6.1	the Parties have agreed in writing the relevant Schedule(s) specifying, as appropriate, the Number Range(s) applying from time to time, or the service is launched pursuant to paragraph 8 of the main body of the Agreement to which such Data Management Amendment order relates;

7.6.2	the Requesting Party has given written confirmation to the Changing Party that the Number Ranges have been allocated to the Requesting Party by the Director General for use by the Requesting Party or the Requesting Party provides written authority, from the Third Party Operator to whom the Number Range has been allocated, to carry out the requested Data Management Amendment;

7.6.3	the Parties have agreed such test facilities as may reasonably be required and the Requesting Party has provided such test facilities (including, without limitation, such number of test lines and appropriate tone/announcement) by the time of commencement of the implementation programme;

7.6.4	the Changing Party has agreed the routing plans submitted by the Requesting Party and the Requesting Party has provided the routing plan reference to the Changing Party prior to commencement of the implementation programme;

7.6.5	the order form has been completed correctly;

and sufficient Capacity (for testing of the Data Management Amendment) is ready for service or has been ordered with a Ready For Service Date being prior to the commencement date of the implementation programme.

7.7	Completion of the implementation programme is subject to the ongoing availability of sufficient test facilities.

7.8	If any of the provisions of paragraphs 7.6 and/or 7.7 above are not being complied with, the Changing Party may suspend the implementation programme and shall advise the Requesting Party that work shall not recommence until:

7.8.1	the Requesting Party gives written confirmation that the provisions of paragraphs 7.6 and/or 7.7 above (as the case may be) have been met; and

7.8.2	the Parties agree any amendments to the implementation programme, such agreement not to be unreasonably withheld or delayed;

and if work does not recommence within 60 Working Days of notification of suspension, the Data Management Amendment order (subject to the Changing Party giving not less than 60 Working Days’ notice) shall be deemed a cancellation by the Requesting Party.

7.9	A revision of the implementation programme in accordance with paragraph 7.8 above may result in the implementation period being extended to such period as is greater than the periods set out in paragraph 7.1 above or provided under paragraph 7.4 above plus the period of delay.

7.10	On completion of the implementation programme, the Changing Party shall give written confirmation to the Requesting Party of full implementation of the Data Management order.

7.11	A reference to a matter being agreed between the Parties means such agreement not to be unreasonably withheld or delayed. If agreement between the Parties is not reached pursuant to paragraph 7.4 either Party may notify the other in writing of a Dispute.