ANNEX C

SCHEDULE 174

BT SCCP Signalling Transit Service

1.	Definitions

1.1	In this Schedule, a reference to a paragraph or Appendix unless stated otherwise, is to a paragraph or Appendix of this Schedule. Words and expressions have the meaning given in Annex D, except as shown below:

“GSM Operator”	an operator authorised to offer GSM Service;

“GSM Service”	mobile telephony services and facilities provided under the GSM specification;

“Nominated Third Party GSM

Operator”	a GSM Operator nominated by the Operator to whom the Operator requests BT to transmit Signalling Messages;

“SCCP Signalling Connection”	a Signalling Link between the Operator System and the BT System using SCCP by which Signalling Messages are transmitted and received by the Operator System to enable an Operator Customer to obtain mobile telephony service from one or more GSM Operators pursuant to arrangements between such GSM Operator(s) and the Operator;

“SCCP”	Signalling Connection Control Part as defined in the publication known as the ITU-T Blue Book Volume VI - Fascicle VI.7 Specifications of Signalling System No 7;

“Signalling Message”	a signalling message as defined in the publication known as the ITU-T Blue Book Volume VI Fascicle VI.7 Specifications of Signalling System No 7 being data necessary to support GSM Roaming;

2.	Description of Service

2.1	Subject to the provisions of this Schedule, at the Operator’s request, BT shall provide a SCCP Signalling Connection and shall convey and deliver to a Nominated Third Party GSM Operator system, via that SCCP Signalling Connection, Signalling Messages handed over from the Operator System destined for that Nominated Third Party GSM Operator system.

2.2	Before requesting a SCCP Signalling Connection, the Operator shall make all necessary arrangements with the Nominated Third Party GSM Operator(s) to exchange Signalling Messages pursuant to this Schedule.

2.3	Any necessary technical requirements for the conveyance of Signalling Messages shall be agreed between the Parties before BT shall be obliged to convey any Signalling Message under paragraph 2.1.

2.4	At the time of requesting a SCCP Signalling Connection, the Operator shall propose its Nominated Third Party GSM Operator(s) by whom Signalling Messages are to be received.

2.5	BT GSM Roaming Service Signalling Messages are not conveyed pursuant to this Schedule.

2.6	A SCCP Signalling Connection provided by BT shall comprise a Signalling Link and shall not comprise a dedicated private circuit.

2.7	BT shall not provide or maintain a SCCP Signalling Connection unless the Operator has a direct connection to a BT ISC.

2.8	For the avoidance of doubt,    BT shall not be obliged to    provide or maintain a SCCP Signalling Connection that    BT reasonably believes could    result in damage to the signalling network of any    Nominated Third Party GSM Operator system.

2.9	BT shall convey Signalling Messages during those periods of time and at the same standard and quality of service as BT conveys such Signalling Messages for the generality of its Customers.

2.10	Each Party shall correct faults which occur in its System which affect the conveyance of Signalling Messages in accordance with such Party’s normal engineering practices. For the avoidance of doubt, neither Party warrants that its System is, or will be, free from faults.

2.11	If there are abnormally high volumes of Signalling Messages that the BT System cannot convey, the Operator may apply network management controls in the Operator System or BT may apply network management controls in the BT System.

2.12	BT shall not be liable for the acts or omissions of any Third Party in respect of any Signalling Message transmitted to, or received from, such Third Party.

3.	Forecasts

3.1	Each quarter, the Operator shall provide to BT a rolling two year forecast of the total number of Signalling Messages received and transmitted (where each Signalling Message shall not exceed 272 octets in length) which the Operator would reasonably expect to transmit to and receive from BT via each SCCP Signalling Connection.

3.2	The Operator shall not exceed its forecasts and, if requested by BT, the Operator shall provide reasonable facilities for BT to ascertain the Operator’s conformance with its obligations at paragraph 3.1.

4.	Routing

4.1	The conveyance of Signalling Messages shall be in accordance with the routing principles specified in Annex A.

5.	Rearrangements

5.1	The Operator may request the rearrangement of any SCCP Signalling Connection in accordance with the following procedure:

5.1.1	the Operator shall submit to BT a written request of the change required and supply any other information reasonably required by BT;

5.1.2	BT shall provide to the Operator a written quotation for any charges payable; and

5.1.3	BT shall carry out the agreed change within a reasonable time after the Operator has accepted the above quotation.

6.	Charging

6.1	The Operator shall pay to BT for each SCCP Signalling Connection the relevant connection charge specified from time to time in the Carrier Price List.

6.2	In addition to the charges payable under sub-paragraph 6.1, the Operator shall pay to BT for each Signalling Message sent the per Signalling Message based usage charges at the rate specified from time to time in the Carrier Price List.

6.3	The charges due under sub-paragraph 6.2 shall be payable monthly in arrears.

6.4	If the Operator requests any rearrangement of a SCCP Signalling Connection pursuant to paragraph 5, any such rearrangement shall be subject to additional charges as specified from time to time in the Carrier Price List.

- Commencement

7.1	BT shall convey Signalling Messages from a date to be agreed in writing by the Parties.