BT STANDARD X.75 INTERCONNECT AGREEMENT

MAIN BODY - APPENDIX A

SPECIFICATION

Contents

X.75 Technical Specification

X.75 TECHNICAL SPECIFICATION

1.	Introduction

This technical specification has been agreed between BT and Operator for the interconnection of their respective Systems.

2.	Address Structure

All network addresses shall conform to ITU-T Recommendation X.121.

3.	Protocol

The protocol used on the internetwork links shall be ITU-T Recommendation X.75 (1984) except where otherwise stated. Significant parameters of the X.75 protocol are listed below together with the setting of each parameter as agreed by BT and Operator and may be subject to further adjustment.

PARAMETER 	SETTING

Alternative Routing	Yes

Level 2 Protocol	LAPB X.75 984

SLP/MLP	SLP

Modulo 8/128	Modulo 8

Link Address	BT = A

Operator = B

T1	3 Seconds

N2	10

N1	1,080

K	7

T3	10 seconds

Logical Channel Group Number	0

Logical Channel Number Range	1 - 4096

Number of Logical channels	64

T21 Call Request Packet time-out	180 Seconds

Selection of Logical Channels	BT: high to low

Operator: low to high

Diagnostic codes in Clear Request	ITU-T X.75 1984

PARAMETER 	SETTING

T23 Clear Request	60 Seconds

Packet Level sequence Numbering	Modulo 8

D bit supported	No

Max Data field length	128 Octets

Diagnostic codes in Reset	ITU-T X.75 1984

T22 Reset Request	60 seconds

Re-transmission of Reset Request	2 repeats

T20 restart Request	60 Seconds

Re-transmission Restart Request	2 repeats

Reset/Restart/Clear Causes	ITU-T X.75 1984

Action on Q bit change	re-set indication

during M bit sequence

M bit supported	Yes

Transit Network ID	Supported

Call ID in Call Request	Yes

Call ID in Call Connect	Yes

Throughput Class	Not supported

Window Size Negotiation	Yes

Window Size	4

Packet Size Negotiation	up to 1024

Maximum Packet Size	1024 octets

Fast Select	No

Inter-network Closed User Group	No

Inter-network Closed User Group O/G Access	No

Reverse Charging	No

Clear on unsupported utilities	Yes

PARAMETER	SETTING

Datagram	No

Utility Marker	No

Call redirection	No

Protocol ID	Yes

Calling address length	14 digits max.

Action on zero entry utility field	Call clear

Action if Clear Request packet	Call clear

contains unsupported facilities

Action if unsupported facilities	Call clear

in Call Request.