BT NETWORK INTERCONNECTION PROVISIONING MANUAL

SECTION 7 – CESSATION OF CAPACITY

# CONTENTS

CONTENTS	2

7.1	INTRODUCTION	3

7.2  PROCEDURES AND TASK OVERVIEW	3

7.2.1 	Preliminary Planning	3

7.2.2	Orders for the Cessation of Capacity	3

7.2.3	Provision of the Cessation of Capacity	3

7.3 DOCUMENT HISTORY	4

ANNEX A - STAGES AND KEY ACTIVITIES	5

Cessation of Interconnect Capacity	5

Key Activities - Descriptions	6

BT process to ensure traffic removal on route ceases – without ACO	8

BT process to ensure traffic removal on route ceases – with ACO	9

## 7.1	INTRODUCTION

This section describes the provisioning activities required for the cessation of interconnect capacity on traffic routes between an OPERATOR switch and the BT network.

## 7.2  PROCEDURES AND TASK OVERVIEW

### 7.2.1 	Preliminary Planning

The need to cease capacity, as identified by analysis of traffic and capacity forecasts, will be discussed at the Technical Review Meeting Forum. As reductions in capacity may have an effect upon the operation of the interconnect both companies should understand the changes and the need to work in co-ordination, before orders are placed.

Particular attention should be given to scenarios where entire traffic routes are to be ceased, with focus on planning for the timely completion of the associated DMAs.

### 7.2.2	Orders for the Cessation of Capacity

Orders will pass from the OPERATOR to the CTC. An example of an Order Form is set out with explanatory notes in Appendix 22 to this Manual. The CTC will deal with its agents within BT to plan the work necessary to fulfill the order. Each operator (both BT and the CP) should only remove the circuits from service at their end of the circuits and carry out the cessation when agreed with the other party.

### 7.2.3	Provision of the Cessation of Capacity

The Stages, Key Activities and Activity Descriptions for the provision of the Cessation of Circuits are shown in Annex A.

## 7.3 DOCUMENT HISTORY

| ISSUE No. | DATE | AUTHOR | SUMMARY OF CHANGES |
| --- | --- | --- | --- |
| 3.4 | 05/11/2009 | Kevin Young | Change to ensure co-ordination of cessation activities between BT and the Operator. |
| 3.3 | 19/05/2003 | Kevin Young | Change to ensure the linkage with associated DMAs is planned for |
| 3.2 | 12/12/2000 | Chris Mitchell | Editorial changes, rectification of errors in page count in Footer. |
| 3.1 | 16/10/2000 | Kevin Young | Conversion of format and section numbering due to rationalisation of ISI and CSI sections. |

# ANNEX A - STAGES AND KEY ACTIVITIES

## Cessation of Interconnect Capacity

## Key Activities - Descriptions

| Activity Name | Activity Descriptions | Owner |
| --- | --- | --- |
| CP sends Firm Order to BT | OPERATOR sends Capacity Order to BT. If the Capacity Order is for removal of an entire Traffic Route the OPERATOR must ensure that all associated DMAs for removal of traffic from the relevant Traffic Route have been completed prior to the submission of such Capacity Order. | OPERATOR sends the Capacity Order to the BT CTC. |
| Order validation by CTC. | The CTC will validate the order against the agreed arrangements for the interconnect contained in the Technical Master Plan. An invalid order will be passed back to the OPERATOR and will not be progressed. | BT CTC |
| Is order for cease of an entire traffic route ? | Both BT and the OPERATOR should be aware of whether the cease order is for cessation of an entire traffic route. The BT CTC should verify whether this is the case during order validation. | BT CTC and OPERATOR |
| Have all associated DMAs and traffic removal (both BT to CP and CP to BT) been completed ? | Both BT and the OPERATOR should be aware of the associated DMAs where an entire traffic route is being ceased. The OPERATOR should ensure that the associated DMAs are complete and that they have removed all CP to BT traffic prior to the submission of the relevant cease orders. | BT CTC and OPERATOR |
| Reject cessation order pending DMA completion | Where cessation orders are submitted for capacity where an entire traffic route is to be ceased and the associated DMAs have not been completed, the BT CTC shall refer the orders back to the CP | BT CTC |
| SRAs issued to BT and OPERATOR | Signalling and Routing Advices for the route are issued by BT and sent out within BT and to the OPERATOR with cessation co-op dates included. | BT CTC |
| Databuild | The OPERATOR and BT amend their Databuild in conjunction to reflect the cessation. | BT and OPERATOR |
| Cease Capacity | The capacity is ceased at both ends. | BT and OPERTOR |
| Issue Removal Certificate | When the circuit has been ceased a Removal certificate is issued. | BT CTC |

## BT process to ensure traffic removal on route ceases – without ACO

## BT process to ensure traffic removal on route ceases – with ACO

END OF SECTION