------------------------------------------------------------

INTERFACE SPECIFICATION FOR THE

INTERCONNECTION OF THE OPERATOR SYSTEM

AND THE BT SYSTEM

-------------------------------------------------------------

GENERIC

LOOP DISCONNECT IN TS16

SIGNALLING

INTERFACE SPECIFICATION

I/C Specification 0110

0     CONTENTS

1. INTRODUCTION	3

1.1 General	3

2. 2. SIGNALLING REPERTOIRE AND CODINGS	4

2.1 Signalling Bits	4

2.2 Miscellaneous Signals	5

3. LOOP DISCONNECT SIGNAL CHARACTERISTICS	5

3.1 Principles	5

3.2 Distortion Limits	5

3.2.1 Pulse Rate	5

3.2.2 Pulse Timings	5

3.2.3 Interdigit Pause (IDP)	5

3.2.4 Pre-sending Pause (PSP)	5

3.2.5 Spurious Signals	6

4. SIGNALLING PROTOCOL REQUIREMENTS	6

4.1 Call Process Flow Diagrams	6

4.2 Seizure	6

4.3 Digit Break	6

4.4 Digit Make	6

4.5 Answer	7

4.6 Clearback	7

4.7 Release Forward	7

4.8 Circuit Free	7

5. CALL HANDLING REQUIREMENTS	7

5.1 Digit Recognition	8

5.2 Equipment Busy	8

5.3 Equipment Free	8

5.3.1 Human Intervention (Bureau Services)	8

6. TONE SPECIFICATION	8

7. RECORDED ANNOUNCEMENTS	8

8. DIGIT TONE MULTI FREQUENCY (DTMF)  TONES	9

9. SAFETY	9

10. HISTORY	9

11. REFERENCES	9

APPENDICES 1-7    LD (TS16) SIGNALLING SEQUENCE

# 1.	INTRODUCTION

## 1.1	General

This specification details the signalling requirements for circuits interconnecting the BT

System, and the Operator System, using 30 channel Pulse Code Modulation (PCM) with Loop Disconnect (LD) type signalling in Timeslot 16.

Signalling Interfaces are assumed for the purposes of this document to be located at Signalling Termination's, (STs).  (See figure 1.)

# 2.	SIGNALLING REPERTOIRE AND CODINGS

## 2.1	Signalling Bits

A 16-Frame Multiframe is used, in accordance with ITU-T Recommendation G.732.  Signalling takes place in Frames 1 to 15.  During each frame two sets of four signalling bits, labelled a b c d, are allocated to carry the signalling information for two of the traffic channels.  These signalling bits are carried in Timeslot 16.

The codes allocated by BT, together with their meanings, are shown in Table 1.

TABLE 1

SIGNALS AND CODINGS

| Bits 1-4/5-8 of Timeslot 16. These sig. codes apply in all frames except frame "0". |  |  |  |  | Signalling Condition in Forward Direction From BT System to the Operator System. | Signalling Condition in Backward Direction From the Operator System to the BT System |
| --- | --- | --- | --- | --- | --- | --- |
| a | b | c | d |  |  |  |
| 1 | 1 | 1 | 1 | Circuit Idle |  | Circuit Busy |
| 0 | 0 | 1 | 1 | Circuit Seized |  | Answer |
| 1 | 0 | 1 | 1 | Dial Break |  | Not Used |
| 0 | 1 | 1 | 1 | Not Used |  | Circuit Free |
| 0 | 0 | 0 | 1 | Operator Override |  | Not Used |
| 1 | 0 | 0 | 1 | Not Used |  | Not Used |
| 1 | 1 | 0 | 1 | Not Used |  | Not Used |
| 0 | 1 | 0 | 1 | Not Used |  | Not Used |
| 0 | 0 | 0 | 0 | Must not be used as signifies multi-frame alignment in frame "0" |  |  |

2.2	Miscellaneous Signals

It should be noted that the repertoire of signals shown in Table 1 is comprehensive. The Operator Override code is a forward signal used to allow an operator to monitor a busy line for caller verification and fault indication purposes.

The receipt of a 'Not Used' coding from the BT System may be regarded as a fault condition by the Operator  System.

# 3.	LOOP DISCONNECT SIGNAL CHARACTERISTICS

## 3.1	Principles

In the BT System numerical address information is transmitted from the Outgoing ST to the Incoming ST in the form of controlled break pulses; with the digit '0' represented by 10 break pulses.  The nominal break/space ratio is 2:1, and the pulse rate a nominal 10 pulses per second.  This results in a break pulse being of a nominal 66.67ms.

There is a Pre-Sending Pause (PSP) prior to sending the first digit, which for the purposes of this specification is defined as the time interval from the sending of a seizure signal by the Outgoing ST, to the sending of the first digit break pulse.  Each digit is separated from the next by an Interdigit Pause (IDP), which is defined as the time interval between the trailing edge of the last break of one digit and the leading edge of the first break of the following digit.

## 3.2	Distortion Limits

### 3.2.1  Pulse Rate

The pulse rate shall be in the range 7-12 pulses per second (pps).

### 3.2.2   Pulse Timings

Break pulses shall normally be of 27ms to 130ms duration.  However, in the event that the calling BT  Customer clears during transmission of a break pulse, the Outgoing BT  ST will be unable to distinguish immediately between the clear and the break pulse.  In these circumstances the break pulse may extend to 225ms duration, followed by transmission of the Circuit Idle code.

Make pulses of BT  originated Calls shall be of 11ms to 120ms duration.

### 3.2.3   Interdigit Pause (IDP)

The IDP shall be in the range of 200-1110ms.  The specific value will be route dependent.

### 3.2.4   Pre-sending Pause (PSP)

The PSP shall be in the range of 50-880ms.  The specific value will be route dependent.

*Note: Both the IDP and PSP may be longer than these values, if the next digit is not available.  (Also see paragraph 5.)

### 3.2.5   Spurious Signals

Spurious break pulses of up to 15ms duration may be generated.  Spurious make (Seize) pulses of up to 5ms duration may be generated and should be ignored.

# 4.	SIGNALLING PROTOCOL REQUIREMENTS

Appendices 1 to 7 show the signalling protocols and inband interactions in diagrammatic form.  These are also described in text in the following sub-paragraphs.  The two should be read in conjunction with one another.

## 4.1	Call Process Flow Diagrams

The Operator shall provide Call flow diagrams for each separate service to BT. These flow diagrams must include details of time-out durations, connection of answer signals, supervisory tones, messages and any other relevant information. The flow diagrams will be subject to agreement with BT and will be  utilised during commissioning and acceptance testing. The Call flow diagrams will then be included  within the Operations and Maintenance Manual.

## 4.2	Seizure

Seize signals from the Outgoing (O/G) termination shall only be validated if the signal being sent by the incoming, (I/C), termination is Circuit Free code 0111.  The Seize signal sent from the O/G termination is a continuous 0011 code.  This signal shall be validated for at least 20ms.  Following validation the Circuit Busy signal 1111 shall be returned.  The total delay, from receipt of the 0011 Seize code, to return of the 1111 code shall not exceed 200ms.

## 4.3	Digit Break

The Digit Break signal 1011 is sent by the distant O/G termination.  The break characteristics shall be in accordance with the parameters specified in paragraph 3.2 Digit breaks lasting less than 16ms shall be ignored.  Digit breaks lasting longer than 27ms shall be accepted as valid by the System.

## 4.4	Digit Make

The Digit Make signal 0011 is sent by the distant O/G termination.  The Make characteristics shall be in accordance with the parameters specified in paragraph 3.2.  Digit Makes lasting less than 6ms shall be ignored.  Digit Makes lasting longer than 10ms shall be accepted as valid by the System.  A Make period longer than 200ms must be taken as an IDP.  A Make period shorter than 120ms must not be taken as an IDP.

Digits may be ignored if enough digits have been received to route the Call.

## 4.5	Answer

An Answer Signal will be returned for all successful Calls including non chargeable (free) Calls. Call failures e.g. congestion and number unavailable will not return the answer signal.

Answer is signified by a continuous 0011 code sent by the I/C termination.  The transition from 1111 to 0011 shall, if possible, be avoided during the receipt of Digit Break, Clear Forward, and 00R signals.

## 4.6	Clearback

Clearback is represented by a continuous 1111 code sent by the I/C termination.  The transition from 0011 to 1111 shall, if possible, be  avoided during the receipt of Digit Break, Clear Forward, and 00R signals.

## 4.7	Release Forward

Prior to Answer Release Forward is represented by either:-

a. A continuous 1111 code, or

b. A period of Dial Break 1011 for 150-225ms followed by a continuous 1111 code.  	(BT  ST only.)

After Answer, Release Forward is represented by a continuous 1111 code.  Release forward shall only be taken as valid if the 1111 code lasts for greater than 20ms.

## 4.8	Circuit Free

Following the receipt of Release Forward the I/C termination shall immediately return the Circuit Busy code 1111 irrespective of the signal it was previously sending.  The Busy Code shall be maintained until the I/C termination is ready to receive the next Call or for 500ms, whichever is the longer.  The I/C termination shall then change the code to the Circuit Free code 0111. The clear down time for the I/C termination shall be a maximum of 1 second from the receipt of release forward.

# 5.	CALL HANDLING REQUIREMENTS

## 5.1	Digit Recognition

## The Operator System will examine the dialled digits received from BT and upon recognising an unallocated sequence of digits connect Number Unobtainable (NU) tone or a recorded announcement to the calling BT circuit. The tone or announcement should be made available for the shortest time of either 20 seconds or when the caller clears. An answer message shall not be returned.

## 5.2	Equipment Busy

If the Operator System is incapable of processing the Call due to blocking within the System, such that the Call could not be processed within 10 seconds, the Operator System shall connect Equipment Engaged Tone (EET) or a recorded announcement to the Calling BT  circuit in accordance with the tone specification given in BS.6305. An Answer message shall not be returned.

## 5.3	Equipment Free

If the Operator System is capable of processing the Call, then Ring Tone  (RT) shall be connected to the calling circuit as specified in BS 6305. An answer message shall be returned for successful calls.

### 5.3.1   Human Intervention (Bureau Services)

If the Call is to be handled by an operator, then upon operator answer, RT shall be removed, and an Answer Signal returned to the BT System as specified in this document. Following the end of conversation, if the calling BT System circuit is still in the seize condition, while the operator has cleared, the Operator System  shall remove the answer signal and restore normal line polarity to the positive and negative wires and shall apply NU tone to the calling BT System 5 seconds after the removal of the answer signal. NU tone shall be applied for a minimum of 20 seconds or until the circuit clears.

# 6.	TONE SPECIFICATION

The Operator System shall generate tones in accordance with BS 6305: 1992, Table 9 for Ring Tone, Equipment Engaged Tone and Number Unobtainable Tone.

# 7.	RECORDED ANNOUNCEMENTS

The maximum peak level of speech for  recorded announcements shall not exceed -8dBm0±3dB.

It is preferred that the Operator System shall always return Recorded Announcements starting at the beginning of an announcement. A minimum of one complete cycle of Recorded Announcement shall be returned to the calling BT customer.

# 8.	DIGIT TONE MULTI FREQUENCY (DTMF)  TONES

In situations where the Operator System requires the use of DTMF tone signalling the tones shall conform to BS 6305: 1982, paragraph 4.4.3.  It shall be noted that BT cannot guarantee the level of DTMF tones when delivered to the Operator System nor can it guarantee the level or clarity of speech messages for onward transmission in store and forward services.

# 9.	SAFETY

Where appropriate the Operator System should comply with BS 6301: 1987

The Operator and BT shall not be responsible for any damage or injuries that are due to induced voltages from lightening or other causes outside of their control.

# 10.	HISTORY

Draft issue 2 March 1995

Draft issue 3 September 1995

Draft issue 4 December 1995

# 11.	REFERENCES

BS 6305 1992

ITU-T G732

END OF SPECIFICATION

Appendices 1-7 follow

Network and Systems

Interconnect Standards

PP224, Angel Centre

403 St Johns Street

LONDON

EC1V  4PL

APPENDIX 1

CIRCUIT SELECTION PHASE

¾¾¾¾¾¾¾¾¾¾® DIRECTION OF CALL SET UP

| ORIGINATING NETWORK NODE | LD IN TS16 SIGNALLING LINK | TERMINATING NETWORK NODE |
| --- | --- | --- |
| IDLE STATE | CIRCUIT IDLE - 1111 ¾¾¾¾¾¾¾¾¾¾® CIRCUIT FREE - 0111 ¬¾¾¾¾¾¾¾¾¾¾ | NOTE: These signals are exchanged continually whilst link is in the idle state. |
| CALL DEMAND RECOGNISED. SELECT FREE AND AVAILABLE CIRCUIT START TIME-OUT TO NON-RECEIPT OF ACKNOWLEDGEMENT | CIRCUIT SEIZED - 0011 ¾¾¾¾¾¾¾¾¾¾® | DETECT & VALIDATE SEIZE SIGNAL ½ ½ ½ ½ ¯ |
| ½ ½ ½ ½ |  | INITIATE CALL DEMAND IN TERMINATING NETWORK ¾¾¾¾¾¾¾¾¾¾® |
| ½ |  |  |
| ½ ½ ½ ¯ STOP TIME-OUT | ACKNOWLEDGEMENT (CIRCUIT BUSY) - 1111 ¬¾¾¾¾¾¾¾¾¾¾ |  |

CIRCUIT SELECTION PHASE COMPLETE

A validation period of at least 20 ms shall be applied by the Operator System.

APPENDIX 2

CALL PATH ESTABLISHMENT PHASE

¾¾¾¾¾¾¾¾¾¾® DIRECTION OF CALL SET-UP

| ORIGINATING NETWORK NODE | LD IN TS16 SIGNALLING LINK | TERMINATING NETWORK NODE |
| --- | --- | --- |
| RECOGNISE DIGIT TO BE SENT |  | NOTE:- CONTINUED FROM APPENDIX 1 |
| ½ TIMED PRE-SENDING ½ PAUSE (PSP) ½ ½ ½ ¯ | CIRCUIT SEIZED - 0011 ¾¾¾¾¾¾¾¾¾¾® CIRCUIT BUSY - 1111 ¬¾¾¾¾¾¾¾¾¾¾ | NOTE:- SENT CONTINUALLY DURING THIS PHASE |
| ½ TIMED INTER-DIGIT ½ PAUSE (IDP) ½ ¯ | FIRST DIGIT * ¾¾¾¾¾¾¾¾¾¾® SECOND DIGIT ¾¾¾¾¾¾¾¾¾¾® |  |
|  | NOTE:- SUBSEQUENT DIGITS AS APPROPRIATE |  |
|  | N-1 DIGIT* ¾¾¾¾¾¾¾¾¾¾® |  |
| ½ TIMED INTER-DIGIT ½ PAUSE (IDP) ¯ | N-1 DIGIT* ¾¾¾¾¾¾¾¾¾¾® | CONNECT CALL ACCORDING TO AGREED PRINCIPLES |
|  | CIRCUIT SEIZED - 0011 ¾¾¾¾¾¾¾¾¾¾® |  |

CALL PATH ESTABLISHMENT PHASE COMPLETE

*SEE APPENDIX 3 FOR DETAILS OF DIGIT SENDING

APPENDIX 3

DIGIT SENDING PHASE

SENDING OF A DIGIT 2

¾¾¾¾¾¾¾¾¾¾® DIRECTION OF CALL SET-UP

| ORIGINATING NETWORK NODE | LD IN TS16 SIGNALLING LINK | TERMINATING NETWORK NODE |
| --- | --- | --- |
| ½ TIMED PRE-SENDING ½ PAUSE (PSP) ½ ½ ¯ | CIRCUIT SEIZED - 0011 ¾¾¾¾¾¾¾¾¾¾® CIRCUIT BUSY - 1111 ¬¾¾¾¾¾¾¾¾¾¾ | NOTE:- SENT CONTINUALLY DURING THIS PHASE |
|  | DIAL BREAK - 1011 ¾¾¾¾¾¾¾¾¾¾® |  |
| ½BREAK PULSE- ½ ½66.67ms (Nominal) ¯ | DIAL BREAK Continued ¾¾¾¾¾¾¾¾¾¾® | DETECT BREAK PULSE ½ ½ ¯ |
| ½MAKE PULSE ½ ½33 ms (nominal) ½ ½ ¯ | DIAL MAKE - 0011 (CIRCUIT SEIZED) ¾¾¾¾¾¾¾¾¾¾® DIAL MAKE continued ¾¾¾¾¾¾¾¾¾¾® | DETECT MAKE PULSE (OR IDP) ½ ¯ |
| ½BREAK PULSE ½ ½66.67ms (Nominal) | DIAL BREAK - 1011 ¾¾¾¾¾¾¾¾¾¾® |  |
| ½ ¯ | DIAL BREAK Continued ¾¾¾¾¾¾¾¾¾¾® | ½DETECT BREAK PULSE ½ ¯ |
| START OF IDP | CIRCUIT SEIZED - 0011 ¾¾¾¾¾¾¾¾¾¾® | VALIDATE IDP (OR MAKE PULSE ½ ¯ |

APPENDIX 4

ANSWER PHASE

¾¾¾¾¾¾¾¾¾¾® DIRECTION OF CALL SET-UP

| ORIGINATING NETWORK NODE | LD IN TS16 SIGNALLING LINK | TERMINATING NETWORK NODE |
| --- | --- | --- |
|  | CIRCUIT SEIZED - 0011 ¾¾¾¾¾¾¾¾¾¾® CIRCUIT BUSY - 1111 ¬¾¾¾¾¾¾¾¾¾¾ | NOTE:- CONTINUED FROM APPENDIX 3 |
| NOTE: - BACKWARD PATH |  | BACKWARD TRANSMISSION PATH PROVIDED |
|  |  | RINGING TONE SENT TO CALLER (WHERE APPROPRIATE) ¬¾¾¾¾¾¾¾¾¾¾ |
| ½DETECT AND ½VALIDATE ANSWER ½ ¯ | ANSWER - 0011 ¬¾¾¾¾¾¾¾¾¾¾ | NOTE:- ANSWER MESSAGE SENT CONTINUALLY WHILST CALL IS IN ANSWERED STATE |
| NOTE: TRANSMISSION OF FURTHER BREAK PULSES INHIBITED |  |  |
| ¬¾¾¾¾¾¾¾¾¾¾ | BOTHWAY TRANSMISSION PATH AVAILABLE | ¾¾¾¾¾¾¾¾¾¾® |

ANSWER PHASE COMPLETE

APPENDIX 5

CALL RELEASE  PHASE

CALLING SUBSCRIBER CLEARS FIRST

¾¾¾¾¾¾¾¾¾¾® DIRECTION OF CALL SET-UP

| ORIGINATING NETWORK NODE | LD IN TS16 SIGNALLING LINK | TERMINATING NETWORK NODE |
| --- | --- | --- |
|  |  | NOTE:- CONTINUED FROM APPENDIX 4 |
| DETECT CALLING SUB CLEAR | CIRCUIT IDLE - 1111 ¾¾¾¾¾¾¾¾¾¾® | DETECT RELEASE FORWARD |
|  | CIRCUIT BUSY - 1111 ¬¾¾¾¾¾¾¾¾¾¾ | MAINTAIN FOR 500ms MINIMUM |
| IDLE STATE | CIRCUIT FREE - 0111 ¬¾¾¾¾¾¾¾¾¾¾ | RELEASE COMPLETE |
| ¬¾¾¾¾¾¾¾¾¾¾ | LINK FREE TO RECEIVE FURTHER CALLS | ¾¾¾¾¾¾¾¾¾¾® |

CALL RELEASE PHASE COMPLETE

APPENDIX 6

CALL RELEASE  PHASE (ANSWERED CALL)

RPE SYSTEM  CLEARS FIRST

¾¾¾¾¾¾¾¾¾¾® DIRECTION OF CALL SET-UP

| ORIGINATING NETWORK NODE | LD IN TS16 SIGNALLING LINK | TERMINATING NETWORK NODE |
| --- | --- | --- |
|  |  | NOTE:- CONTINUED FROM APPENDIX 5 |
| START CSH TIME OUT | CIRCUIT BUSY - 1111 ¾¾¾¾¾¾¾¾¾¾® | OPERATOR SYSTEM TIME OUT MATURES |
| 1. CSH time Out Matures |  |  |
| APPLY FORWARD RELEASE | CIRCUIT IDLE - 1111 ¾¾¾¾¾¾¾¾¾¾® | CIRCUIT BUSY MAINTAINED UNTIL RPE SYSTEM IS READY TO RECEIVE A NEW CALL |
| CALLER RECEIVES TONE OR ANNOUNCEMENT |  |  |
| IDLE STATE | CIRCUIT FREE - 0111 ¬¾¾¾¾¾¾¾¾¾¾ |  |
|  | CALL RELEASE COMPLETE |  |
| 2. Calling Sub Clears |  |  |
| DETECT CALLING SUB CLEAR | CIRCUIT IDLE - 1111 ¾¾¾¾¾¾¾¾¾¾® | CIRCUIT BUSY MAINTAINED UNTIL RPE SYSTEM IS READY TO RECEIVE A NEW CALL |
| IDLE | CIRCUIT FREE - 0111 ¬¾¾¾¾¾¾¾¾¾¾ |  |

CALL RELEASE COMPLETE

NOTE:- 1 and 2 are alternatives.

APPENDIX 7

CALL SEQUENCE  FOR UNANSWERED CALL

¾¾¾¾¾¾¾¾¾¾® DIRECTION OF CALL SET-UP

| ORIGINATING NETWORK NODE | LD IN TS16 SIGNALLING LINK | TERMINATING NETWORK NODE |
| --- | --- | --- |
|  | CIRCUIT SEIZED - 0011 ¾¾¾¾¾¾¾¾¾¾® |  |
|  | ACKNOWLEDGEMENT (CIRCUIT BUSY) - 1111 ¬¾¾¾¾¾¾¾¾¾¾ |  |
|  | DIGITS ¾¾¾¾¾¾¾¾¾¾® |  |
| DIRECT CALLING SUB CLEAR | CIRCUIT IDLE - 1111 ¾¾¾¾¾¾¾¾¾¾® |  |
|  |  | CIRCUIT BUSY MAINTAINED UNTIL RPE SYSTEM IS READY TO RECEIVE A NEW CALL |
|  | CIRCUIT FREE - 0111 ¬¾¾¾¾¾¾¾¾¾¾ |  |

CALL RELEASE COMPLETE